Mt. Morgan SITE PROCEDURE

Plant Sampling and Metallurgical Data Collection

Standard Work Procedure  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | HM-PRC-VXX-PRO205 |
| --- | --- |
| Title | Plant Sampling and Metallurgical Data Collection |
| Scope | All manual process sampling across the plant, from the scrubber discharge through to final tailings and return water, plus reagent strength sampling, compositing and online analyser verification. |
| Register area | 201 Plant Sampling, on tabs JSEA_MET and SWI-MET |
| Hazard analyses beneath | 25 (JSEA-PRO-MET-201-001 to -025) |
| Work instructions beneath | 25 |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev A |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This procedure covers all manual process sampling across the plant, from the scrubber discharge through to final tailings and return water, plus reagent strength sampling, compositing and online analyser verification. at the Mt. Morgan processing plant. It is the procedure that the 25 hazard analyses and 25 work instructions in register area 201 sit under.

All manual process sampling across the plant, from the scrubber discharge through to final tailings and return water, plus reagent strength sampling, compositing and online analyser verification.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician / Met Lab Technician | Carry out the task to the schedule and to the work instruction. Only carry out work where competent and authorised to do so. Record what was actually done. Work not done is recorded as not done - never invented and never back-filled. Stop and report if any control in the JSEA is not in place. |
| Shift Supervisor / Coordinator | Authorise the task and confirm the area is fit before it starts. Know that the task is in progress and where the person is. Understand that much of the plant is automated and equipment may start or stop without warning. Act on any off-specification result reported from the task. |
| Metallurgist | Set and maintain the schedule - what is done, how often, and what is reported. Review the results and act on the trends. Investigate any variance outside the agreed tolerance before a period is closed. |
| Process Manager / Superintendent | Ensure all personnel are trained and competent. Ensure this procedure and the documents beneath it are reviewed at the stated interval. Approve the risk ranking in the JSEAs. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| JSEA | Job safety and environmental analysis - one per task, giving the step-by-step hazards, controls and risk ranking. |
| SWI | Standard work instruction - one per task, giving the step-by-step method. |
| SWP | Standard work procedure - this document. |
| Steady state | Normal operation held for at least 15 to 20 minutes. |
| ReCYN | The resin-in-pulp cyanide and metal recovery process in Areas 041, 051, 071 and 081. |
| CN Free / WAD CN | Free cyanide by silver nitrate titration; weak acid dissociable cyanide by distillation. |

# 4  POTENTIAL HAZARDS

The step-by-step hazard analysis for each task, with the risk ranking, is in the JSEA for that task. The hazards common across this area are:

- Hydrogen cyanide gas released at the open sample point

- Splash of cyanide-bearing slurry or solution to skin or eyes

- Contact with an agitator, sample cutter, pump or screen that starts without warning

- Hearing damage in the mill, screen, compressor and blower areas

- Slip, trip or fall on wet or slurry-covered walkways around sample points

- Manual handling injury from carrying sample buckets, containers and composite drums

- Non-representative sample leading to a wrong process decision

- Cross-contamination from a container or cutter not cleaned between points

- Fall from a tank top, launder platform, column platform or access ladder

- Working alone in a cyanide area where a collapse would not be seen

- Confined space atmosphere at a tank top hatch, launder or hopper opening

- Contact with hot solution or hot surface at the elution, stripping or regeneration circuits

- Release from a pressurised sample line, hose or fitting

- Mercury vapour in the gold room, elution and retort areas

# 5  PPE

Standard PPE is worn for all normal operations: hard hat, safety glasses, high-visibility long-sleeved shirt and long trousers, safety boots, and gloves appropriate to the task.

The following is added where the task requires it:

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Hearing protection

- Harness and fall arrest where no fixed platform exists

- Heat-resistant gloves

- Face shield

- Acid-resistant gloves, apron and face shield

- Chemical splash goggles

- Personal multi-gas monitor including H2S, calibrated and in test date

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

6.2  Procedure and implementation steps

The documents below are the controlled method for each task. This procedure does not repeat them.

| JSEA | Work instruction | Task | Class |
| --- | --- | --- | --- |
| JSEA-PRO-MET-201-001 | SWI-PRO-MET-201-001 | Front-End Sampling Round - Scrubber Discharge Hopper and Trash Screen | Operating |
| JSEA-PRO-MET-201-002 | SWI-PRO-MET-201-002 | IsaMill Feed and Discharge Sampling Round | Operating |
| JSEA-PRO-MET-201-003 | SWI-PRO-MET-201-003 | Leach Tank Sampling Round - 031-TK-001 Launder and Discharge | Safety-critical |
| JSEA-PRO-MET-201-004 | SWI-PRO-MET-201-004 | CIL Sampling Round - 032-TK-002 to 032-TK-006 Launders | Safety-critical |
| JSEA-PRO-MET-201-005 | SWI-PRO-MET-201-005 | Carbon Concentration Measurement in CIL Tanks | Safety-critical |
| JSEA-PRO-MET-201-006 | SWI-PRO-MET-201-006 | Loaded Carbon Sampling at Loaded Carbon Screen 032-SC-002 | Safety-critical |
| JSEA-PRO-MET-201-007 | SWI-PRO-MET-201-007 | Barren Carbon Sampling at Carbon Dewatering Screen 061-SC-013 | Safety-critical |
| JSEA-PRO-MET-201-008 | SWI-PRO-MET-201-008 | CIL Tail Sampling at Carbon Safety Screen 032-SC-003 Undersize | Safety-critical |
| JSEA-PRO-MET-201-009 | SWI-PRO-MET-201-009 | Metal Adsorption Sampling Round - 041-TK-007 and 041-TK-008 (ReCYN) | Safety-critical |
| JSEA-PRO-MET-201-010 | SWI-PRO-MET-201-010 | Cyanide Adsorption Sampling Round - 051-TK-009 and 051-TK-010 (ReCYN) | Safety-critical |
| JSEA-PRO-MET-201-011 | SWI-PRO-MET-201-011 | Resin Concentration Measurement in Adsorption Tanks | Safety-critical |
| JSEA-PRO-MET-201-012 | SWI-PRO-MET-201-012 | Loaded and Barren Resin Sampling at Screens and Hoppers | Safety-critical |
| JSEA-PRO-MET-201-013 | SWI-PRO-MET-201-013 | Gold Elution, Electrowinning and Pre-Soak Solution Sampling | Safety-critical |
| JSEA-PRO-MET-201-014 | SWI-PRO-MET-201-014 | Copper Elution, Reactor and Thickener Sampling | Safety-critical |
| JSEA-PRO-MET-201-015 | SWI-PRO-MET-201-015 | Copper Filtrate and Precipitate Sampling | Safety-critical |
| JSEA-PRO-MET-201-016 | SWI-PRO-MET-201-016 | Cyanide Elution, Stripping and AVR Sampling | Safety-critical |
| JSEA-PRO-MET-201-017 | SWI-PRO-MET-201-017 | Tails Thickener Feed, Overflow and Underflow Sampling | Safety-critical |
| JSEA-PRO-MET-201-018 | SWI-PRO-MET-201-018 | Final Tailings and TSF Return Water Sampling | Safety-critical |
| JSEA-PRO-MET-201-019 | SWI-PRO-MET-201-019 | Water Treatment Plant Feed and Discharge Sampling | Safety-critical |
| JSEA-PRO-MET-201-020 | SWI-PRO-MET-201-020 | Reagent Solution Strength Sampling - NaCN, NaOH, H2SO4, NaCl, Lime and NaSH | Safety-critical |
| JSEA-PRO-MET-201-021 | SWI-PRO-MET-201-021 | Composite Sample Build-Up, Splitting and Handover to Laboratory | Operating |
| JSEA-PRO-MET-201-022 | SWI-PRO-MET-201-022 | Online Analyser Verification Sampling - pH, Cyanide and Dissolved Oxygen | Safety-critical |
| JSEA-PRO-MET-201-023 | SWI-PRO-MET-201-023 | Sample Point Access, Isolation, Flushing and Housekeeping | Safety-critical |
| JSEA-PRO-MET-201-024 | SWI-PRO-MET-201-024 | Sampling During Start-Up, Shutdown and Upset Conditions | Safety-critical |
| JSEA-PRO-MET-201-025 | SWI-PRO-MET-201-025 | Sampling Under Cyanide Alarm or Restricted Access Conditions | Safety-critical |

6.3  Schedule

| Task | Frequency |
| --- | --- |
| 201-001 Front-End Sampling Round - Scrubber Discharge Hopper and Trash Screen | Hourly spot sample, composited to four 12-litre composites per shift. |
| 201-002 IsaMill Feed and Discharge Sampling Round | Hourly spot samples on feed and discharge, composited per shift. Full sizing at least once per shift. |
| 201-003 Leach Tank Sampling Round - 031-TK-001 Launder and Discharge | Hourly spot sample, composited to four 12-litre composites per shift. |
| 201-004 CIL Sampling Round - 032-TK-002 to 032-TK-006 Launders | Once per shift across all five tanks; CIL-1 and CIL-5 hourly. |
| 201-005 Carbon Concentration Measurement in CIL Tanks | Once per shift on every tank. |
| 201-006 Loaded Carbon Sampling at Loaded Carbon Screen 032-SC-002 | Every carbon transfer, and at least once per shift while transfers are running. |
| 201-007 Barren Carbon Sampling at Carbon Dewatering Screen 061-SC-013 | Every elution batch, on the barren carbon returning to CIL. |
| 201-008 CIL Tail Sampling at Carbon Safety Screen 032-SC-003 Undersize | Hourly spot sample, composited per shift. Carbon check on every sample. |
| 201-009 Metal Adsorption Sampling Round - 041-TK-007 and 041-TK-008 (ReCYN) | Once per shift on each tank; more often during commissioning. |
| 201-010 Cyanide Adsorption Sampling Round - 051-TK-009 and 051-TK-010 (ReCYN) | Once per shift on each tank; hourly during commissioning and after any upset. |
| 201-011 Resin Concentration Measurement in Adsorption Tanks | Once per shift on every adsorption tank. |
| 201-012 Loaded and Barren Resin Sampling at Screens and Hoppers | Every resin transfer, both directions. |
| 201-013 Gold Elution, Electrowinning and Pre-Soak Solution Sampling | Each elution batch - pre-soak, mid-strip, end-of-strip, and electrowinning feed and barren each shift the cells are running. |
| 201-014 Copper Elution, Reactor and Thickener Sampling | Each elution batch on the columns; hourly on the reactor and thickener while precipitating. |
| 201-015 Copper Filtrate and Precipitate Sampling | Every filter press cycle on the cake; each shift on the filtrate. |
| 201-016 Cyanide Elution, Stripping and AVR Sampling | Each elution and stripping batch; scrubber solution each shift. |
| 201-017 Tails Thickener Feed, Overflow and Underflow Sampling | Hourly on overflow and underflow; each shift on feed. |
| 201-018 Final Tailings and TSF Return Water Sampling | Each shift on final tailings; daily on return water, or as the licence requires. |
| 201-019 Water Treatment Plant Feed and Discharge Sampling | To be set once the circuit narrative is available. |
| 201-020 Reagent Solution Strength Sampling - NaCN, NaOH, H2SO4, NaCl, Lime and NaSH | Every batch made up, and once per shift on the standing solutions. |
| 201-021 Composite Sample Build-Up, Splitting and Handover to Laboratory | Continuous through the shift, with handover at the end of each composite period. |
| 201-022 Online Analyser Verification Sampling - pH, Cyanide and Dissolved Oxygen | Once per shift on each analyser, and after any calibration or maintenance on it. |
| 201-023 Sample Point Access, Isolation, Flushing and Housekeeping | Every time a sample point is used. |
| 201-024 Sampling During Start-Up, Shutdown and Upset Conditions | Whenever the plant is outside steady-state operation. |
| 201-025 Sampling Under Cyanide Alarm or Restricted Access Conditions | Whenever a gas alarm sounds or access is restricted. |

# 7  FLOWCHART

Not reproduced here. Each work instruction carries its own step sequence in Section 6.

# 8  TRAINING

Every person carrying out a task in this area is trained and signed off against the work instruction for that task, and has signed on to the matching JSEA at a communication session.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations, instruments and calculations used in the task.

- Refresher at the interval set by the site training matrix, and after any change to the instruction.

# 9  REFERENCE

- Mt. Morgan site procedure 1. Scrubber Mill Discharge Sampling [LIVE]

- 4034-PR-PRO-002 Sampling Protocol

- 4034-002-VE-025 Rev A McLanahan rotary scrubber IO&M (021-ML-001), received 11 Aug 2026

- 4034-004-VE-025 Rev B Goldquip vibrating screen IOM (021-SC-001), received 11 Aug 2026

- HM-PRC-V01-PRO002 Scrubber and Trash Screening Circuit

- Mt. Morgan site procedures 2. ISAMILL Feed Sampling and 3. ISAMILL Discharge Sampling [LIVE]

- 4034-001-VE-041 Rev A IsaMill Generic Operating Manual and 4034-001-VE-042 Rev A Maintenance Manual, received 11 Aug 2026

- 4034-001-VE-038 Rev 0 IsaMill Control Philosophy / PCS control logic specification, received 11 Aug 2026

- HM-PRC-V01-PRO003 Milling Circuit - IsaMill

- Mt. Morgan site procedure 4. Leach Tank Sampling [LIVE]

- KBK-MIR-MP-PRO-MET-SOP-0048 Pengambilan Sampel dan Pengukuran Profil Tangki Leaching, received 11 Aug 2026

- KBK-MIR-MP-PRO-MET-SOP-0046 Pengukuran Oksigen Terlarut and SOP-0045 Pengukuran Densitas Lumpur, received 11 Aug 2026

- 4034-044-VE-003 Rev B Glencore Hypersparge supersonic gas sparger instructions, received 11 Aug 2026

- HM-PRC-V01-PRO004 Leach Circuit

- Mt. Morgan site procedure 6. CIL-1 & 5 Tank Sampling [LIVE]

- 4034-PR-PRO-002 Sampling Protocol, CIL table

- HM-PRC-V01-PRO005 Carbon in Leach (CIL) Circuit

- Mt. Morgan site procedure 5. Carbon Concentration Sampling [LIVE]

- KBK-MIR-MP-PRO-OPE-SOP-0013 Particle Size Distribution of Activated Carbon

- 4034-PR-PRO-002 Sampling Protocol, CIL table item 3

- 4034-004-VE-025 Rev B Goldquip vibrating screen IOM, received 11 Aug 2026

- 4034-PR-PRO-002 Sampling Protocol, CIL table item 4

- 4034-013-VE-028 Rev A Applied Heat Thermomat 2300 O&M manual, received 11 Aug 2026

- HM-PRC-V01-PRO012 Carbon Regeneration and Kiln Operation

- 4034-PR-PRO-002 Sampling Protocol item 5 - the sample point is still marked (TBC) in the protocol

- 4034-004-VE-025 Rev B Goldquip vibrating screen IOM (032-SC-003), received 11 Aug 2026

- RA-02 Carbon Contamination Register Rev A

- Mt. Morgan site procedure 8. Metal Adsorption 1 & 2 Tank Sampling [LIVE]

- KBK-MIR-MP-PRO-MET-SOP-0049 Pengambilan Sampel dan Pengukuran Profil Tangki Resin, received 11 Aug 2026

- KBK SOP-PROC-CNREC-01 Resin transfer between adsorption tanks, received 11 Aug 2026

- 4034-004-VE-025 Rev B Goldquip vibrating screen IOM (041-SC-004, 041-SC-016), received 11 Aug 2026

- HM-PRC-V01-PRO006 Metal Adsorption Circuit and Resin Transfer

- Mt. Morgan site procedure 14. Cyanide Adsorption 1 & 2 Tank Sampling [LIVE]

- KBK-MIR-MP-PRO-MET-SOP-0049, received 11 Aug 2026

- KBK SOP-PROC-CNREC-01 and CNREC-12 Fresh resin conditioning, received 11 Aug 2026

- 4034-004-VE-025 Rev B Goldquip vibrating screen IOM (051-SC-005/006/020), received 11 Aug 2026

- HM-PRC-V01-PRO007 Cyanide Adsorption Circuit (ReCYN) and Resin Transfer

- Mt. Morgan site procedure 7. Resin Concentration Sampling [LIVE]

- KBK-MIR-MP-PRO-MET-SOP-0058 Distribusi Ukuran Partikel Resin, received 11 Aug 2026

- RA-01 Resin Loss Register Rev A and MM-4034-033-RES Resin Line Schedule (268 lines)

- Mt. Morgan site procedure 9. Metal Elution Sampling [LIVE] - covers part of this task

- KBK SOP-PROC-CNREC-01, -04, -06 and -07 resin transfer procedures, received 11 Aug 2026

- KBK-MIR-MP-PRO-MET-SOP-0049 and SOP-0058, received 11 Aug 2026

- 4034-PR-PRO-002 Sampling Protocol, Gold Recovery table

- 4034-013-VE-028 Rev A Applied Heat Thermomat 2300 O&M manual (061-BR-001, 061-HE-001, 061-HX-001/002, 061-PP-022, 061-TK-060), received 11 Aug 2026

- HM-PRC-V01-PRO010 Gold Elution Pre-Soak Mixing, PRO011 Gold Elution, PRO013 Gold Electrowinning

- Mt. Morgan site procedures 10. Copper Reactor Tank Sampling and 11. Copper Thickener Underflow Sampling [LIVE]

- KBK SOP-PROC-CNREC-05 Metal elution and CNREC-09 Filtration of copper precipitation, received 11 Aug 2026

- 4034-017-VE-096 Rev B Goldquip fabricated ReCYN column IOM (071-CL-002/003), received 11 Aug 2026

- 4034-024-VE-061 Rev 0 AirEng cyanide scrubbing column No.1 exhaust fan IOM (071-FA-008), received 11 Aug 2026

- HM-PRC-V01-PRO014, PRO015 and PRO016

- Mt. Morgan site procedure 12. Copper Filtrate Tank Sampling [LIVE]

- KBK SOP-PROC-CNREC-09 Filtration of copper precipitation and CNREC-10 Release of precipitated copper, received 11 Aug 2026

- HM-PRC-V01-PRO018 Copper Precipitate Filtering

- Mt. Morgan site procedures 13, 15, 16, 17 and 18 - cyanide circuit sampling [LIVE]

- KBK SOP-PROC-CNREC-13 Sampling for the cyanide elution, stripping and scrubbing circuit, received 11 Aug 2026 - a worked procedure for this exact duty

- KBK SOP-PROC-CNREC-02 Cyanide elution and CNREC-03 Cyanide stripping and scrubbing, received 11 Aug 2026

- 4034-017-VE-096 Rev B Goldquip ReCYN column IOM (081-CL-005/006), received 11 Aug 2026

- HM-PRC-V01-PRO019 and PRO020

- Mt. Morgan site procedure 23. Slurry Tail Sampling [LIVE]

- Tail Thickener Field Operator Procedure

- 4034-018A-VE-007 Rev 0 Roytec tails thickener functional specification and control philosophy (051-TH-006), received 11 Aug 2026

- 4034-018A-VE-045 Rev A Roytec 25 m tailings thickener IOM, received 11 Aug 2026

- 4034-019B-VE-007 Rev A Roytec Model 05 continuous flocculant plant control philosophy, received 11 Aug 2026

- HM-PRC-V01-PRO008 Tails Thickener and Tailings Disposal

- 4034-PR-PRO-002 Sampling Protocol, Reagent Area table - TSF solution

- No source held

- Mt. Morgan site procedures 19. Sodium Cyanide Solution, 20. Sodium Cyanide Briquettes, 21. Diluted Sulphuric Acid and 22. Diluted Sodium Hydroxide Sampling [LIVE]

- KBK-MIR-MP-PRO-MET-SOP-0009 caustic soda titration, SOP-0010 cyanide titration, SOP-0025 and SOP-0039/0040 hydrochloric acid titration, SOP-0041/0042 sulphuric acid titration and SOP-0043 AgNO3 standard preparation - the last six received 11 Aug 2026

- KBK-MIR-MP-PRO-OPE-SOP-0076 Fasilitas Cyanide, received 11 Aug 2026

- 4034-046-VE-019 Rev A Transmin quicklime slaking plant functional description, received 11 Aug 2026

- HM-PRC-V01-PRO021 to PRO027

- 4034-PR-PRO-002 Sampling Protocol - 4-hourly to 12-hour shift compositing

- KBK-MIR-MP-PRO-MET-SOP-0036 Preparasi Sampel Komposit and SOP-0035 Preparasi Sampel Leach Profile, both received 11 Aug 2026

- KBK-MIR-LAB-PRE-SOP-001 Sample receipt and PRE-SOP-002 Sample sorting

- Control Philosophy 3.1.x - 243 analog loops including pH, CN and DO analysers

- KBK-MIR-MP-PRO-MET-SOP-0033 Penentuan Konsentrasi Sianida dengan Kolorimetri, SOP-0053 potassium chloride solution for the cyanide analyser, SOP-0030 DO meter calibration and SOP-0031 pH meter calibration - all received 11 Aug 2026

- MM-4034-033-P-09 Tag Register (942 tags)

- KBK-MIR-MP-PRO-MET-SOP-0047 Pengukuran Profil Lumpur Cyclone Over Flow, SOP-0048 leach tank sampling and profile and SOP-0049 resin tank sampling and profile - all received 11 Aug 2026, each carrying the sample point access and flushing steps for its own point

- Mt. Morgan site sampling procedures 1 to 23 [LIVE] - common flushing and cleaning practice

- HM-PRC-V01-PRO030 Whole-of-Plant Start-up and PRO031 Whole-of-Plant Shutdown

- The 24 sequence flowcharts held, of which only 2 are stop sequences

- Written from first principles against the circuit procedures - no direct source held

- SWI-022 Gas Detector Alarm Response (HCN and H2S), issued Rev A

- Control Philosophy 3.1.x - 20 gas detectors at 5 ppm alarm and 10 ppm high-high

# 10  REVIEW

This procedure and the documents beneath it are reviewed every 12 months, and immediately after any incident, plant change, equipment change or change to the schedule.

# 11  REGISTER TRACEABILITY

The three-digit suffix is shared between the two registers, so SWI-PRO-MET-201-011 is the work instruction for JSEA-PRO-MET-201-011.

Registers: HM-MMM-WHS-REG-JSEA-PRO-updated_10.xlsx tab JSEA_MET, and HM-MMM-WHS-REG-XXX-R00-SWI-SWMS.xlsx tab SWI-MET. GGT working masters are MM-4034-033-OR-03-JSEA-Register-RevE.xlsx and MM-4034-033-OR-03-SWI-Register-RevE.xlsx.

| Item | Count |
| --- | --- |
| Register lines in area 201 | 25 |
| Hazard analyses issued | 25 |
| Work instructions issued | 25 |
| Desk-based tasks with a JSEA but no work instruction | 0 |
| Safety-critical | 22 |
| Issued but NOT FOR USE | 1 |