Mt. Morgan SITE PROCEDURE

Metallurgical Accounting and Reconciliation

Standard Work Procedure  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | HM-PRC-VXX-PRO206 |
| --- | --- |
| Title | Metallurgical Accounting and Reconciliation |
| Scope | Metallurgical accounting and reconciliation - daily and monthly mass balance, circuit inventory and lock-up, carbon and resin movement accounting, reagent consumption, bullion and concentrate reconciliation, and the investigation of a balance that does not close. |
| Register area | 202 Metallurgical Accounting, on tabs JSEA_MET and SWI-MET |
| Hazard analyses beneath | 12 (JSEA-PRO-MET-202-001 to -012) |
| Work instructions beneath | 0 |
| Process owner | Heritage Mineral Operations · Business unit Mt. Morgan |
| Document owner | Process Operations Superintendent |
| Document approver | Process Manager |
| Revision | Rev A |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  PURPOSE / SCOPE

This procedure covers metallurgical accounting and reconciliation at the Mt. Morgan processing plant. It is the procedure that the 12 hazard analyses and 0 work instructions in register area 202 sit under.

Metallurgical accounting and reconciliation - daily and monthly mass balance, circuit inventory and lock-up, carbon and resin movement accounting, reagent consumption, bullion and concentrate reconciliation, and the investigation of a balance that does not close.

# 2  RESPONSIBILITIES

| Role | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician / Met Lab Technician | Carry out the task to the schedule and to the work instruction. Only carry out work where competent and authorised to do so. Record what was actually done. Work not done is recorded as not done - never invented and never back-filled. Stop and report if any control in the JSEA is not in place. |
| Shift Supervisor / Coordinator | Authorise the task and confirm the area is fit before it starts. Know that the task is in progress and where the person is. Understand that much of the plant is automated and equipment may start or stop without warning. Act on any off-specification result reported from the task. |
| Metallurgist | Set and maintain the schedule - what is done, how often, and what is reported. Review the results and act on the trends. Investigate any variance outside the agreed tolerance before a period is closed. |
| Process Manager / Superintendent | Ensure all personnel are trained and competent. Ensure this procedure and the documents beneath it are reviewed at the stated interval. Approve the risk ranking in the JSEAs. |

# 3  ABBREVIATIONS / DEFINITIONS

| Term | Meaning |
| --- | --- |
| JSEA | Job safety and environmental analysis - one per task, giving the step-by-step hazards, controls and risk ranking. |
| SWI | Standard work instruction - one per task, giving the step-by-step method. |
| SWP | Standard work procedure - this document. |
| Steady state | Normal operation held for at least 15 to 20 minutes. |
| ReCYN | The resin-in-pulp cyanide and metal recovery process in Areas 041, 051, 071 and 081. |
| CN Free / WAD CN | Free cyanide by silver nitrate titration; weak acid dissociable cyanide by distillation. |

# 4  POTENTIAL HAZARDS

The step-by-step hazard analysis for each task, with the risk ranking, is in the JSEA for that task. The hazards common across this area are:

- An accounting or reconciliation error that hides a real metal loss

- Non-representative sample leading to a wrong process decision

- Exposure to a gold security incident, and to suspicion in one

- Mercury vapour in the gold room, elution and retort areas

- Hydrogen cyanide gas released at the open sample point

- Fall from a tank top, launder platform, column platform or access ladder

- Manual handling injury from carrying sample buckets, containers and composite drums

- Cross-contamination from a container or cutter not cleaned between points

# 5  PPE

Standard PPE is worn for all normal operations: hard hat, safety glasses, high-visibility long-sleeved shirt and long trousers, safety boots, and gloves appropriate to the task.

The following is added where the task requires it:

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Harness and fall arrest where no fixed platform exists

# 6  PROCEDURE

6.1  Introduction

Warning: many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times.

6.2  Procedure and implementation steps

The documents below are the controlled method for each task. This procedure does not repeat them.

| JSEA | Work instruction | Task | Class |
| --- | --- | --- | --- |
| JSEA-PRO-MET-202-001 | - | Daily Metallurgical Accounting and Plant Mass Balance | Operating |
| JSEA-PRO-MET-202-002 | - | Shift and Daily Production Data Entry and Validation | Operating |
| JSEA-PRO-MET-202-003 | - | Monthly Metal Accounting and Reconciliation | Operating |
| JSEA-PRO-MET-202-004 | - | Gold and Silver Circuit Inventory and Lock-Up Reconciliation | Safety-critical |
| JSEA-PRO-MET-202-005 | - | Carbon Inventory and Carbon Movement Accounting | Operating |
| JSEA-PRO-MET-202-006 | - | Resin Inventory and Resin Movement Accounting | Operating |
| JSEA-PRO-MET-202-007 | - | Reagent Consumption Tracking and Reconciliation | Operating |
| JSEA-PRO-MET-202-008 | - | Bullion Reconciliation Against Refinery Outturn | Safety-critical |
| JSEA-PRO-MET-202-009 | - | Copper Concentrate Lot Accounting and Dispatch Reconciliation | Operating |
| JSEA-PRO-MET-202-010 | - | Sample Dispatch, Umpire Assay and External Laboratory Reconciliation | Operating |
| JSEA-PRO-MET-202-011 | - | Metallurgical Report Preparation and Distribution | Operating |
| JSEA-PRO-MET-202-012 | - | Investigation and Correction of Mass Balance Discrepancy | Operating |

6.3  Schedule

| Task | Frequency |
| --- | --- |
| 202-001 Daily Metallurgical Accounting and Plant Mass Balance | Daily, closed before the morning production meeting. |
| 202-002 Shift and Daily Production Data Entry and Validation | Every shift, at handover. |
| 202-003 Monthly Metal Accounting and Reconciliation | Monthly, within the agreed reporting deadline. |
| 202-004 Gold and Silver Circuit Inventory and Lock-Up Reconciliation | Monthly at the close, and after any circuit shutdown or major upset. |
| 202-005 Carbon Inventory and Carbon Movement Accounting | Every carbon movement, reconciled monthly. |
| 202-006 Resin Inventory and Resin Movement Accounting | Every resin transfer, reconciled monthly. |
| 202-007 Reagent Consumption Tracking and Reconciliation | Daily entry, reconciled monthly against deliveries and stock. |
| 202-008 Bullion Reconciliation Against Refinery Outturn | Every dispatch, reconciled at the monthly close. |
| 202-009 Copper Concentrate Lot Accounting and Dispatch Reconciliation | Every lot. |
| 202-010 Sample Dispatch, Umpire Assay and External Laboratory Reconciliation | Every dispatch; reconciliation monthly. |
| 202-011 Metallurgical Report Preparation and Distribution | Daily, weekly and monthly per the reporting calendar. |
| 202-012 Investigation and Correction of Mass Balance Discrepancy | Whenever a balance falls outside the agreed tolerance. |

# 7  FLOWCHART

Not reproduced here. Each work instruction carries its own step sequence in Section 6.

# 8  TRAINING

Every person carrying out a task in this area is trained and signed off against the work instruction for that task, and has signed on to the matching JSEA at a communication session.

- Site induction, current.

- Cyanide awareness and gas detection competency where the area requires it.

- Working at height competency where the task requires it.

- Competency in the determinations, instruments and calculations used in the task.

- Refresher at the interval set by the site training matrix, and after any change to the instruction.

# 9  REFERENCE

- No direct source held. Written from first principles against the plant configuration.

- MM-4034-033-P-09 Tag Register (942 tags) and the mechanical equipment list

- 4034-PR-PRO-002 Sampling Protocol - the sample points the balance is built on

- [LIVE] SOP-Mt.Morgan-Data Entry-xxx.docx and its change form template

- Mt. Morgan Changeform Template

- No direct source held. Written from first principles.

- RA-01 Resin Loss Register and RA-02 Carbon Contamination Register, both Rev A

- Mechanical equipment list - vessel volumes

- MM-4034-033-RES Resin Line Schedule (268 lines)

- KBK-MIR-MP-PRO-MET-SOP-0044 Pengiriman Bullion, received 11 Aug 2026

- RA-02 Carbon Contamination Register Rev A and its study report

- Mechanical equipment list

- KBK-MIR-MP-PRO-OPE-SOP-0013 Particle Size Distribution of Activated Carbon

- RA-01 Resin Loss Register Rev A and its study report

- KBK-MIR-MP-PRO-MET-SOP-0058 Distribusi Ukuran Partikel Resin and KBK SOP-PROC-CNREC-12 Fresh resin conditioning, both received 11 Aug 2026

- HM-PP-006A Reagent First Fill and HM-PP-006B Reagents Consumption 2026

- Mt. Morgan Reagent Consumption and Pricing folder, including the pricing reference set

- HM-PRC-V01-PRO021 to PRO027 reagent procedures

- KBK-MIR-MP-PRO-MET-SOP-0044 Pengiriman Bullion, received 11 Aug 2026 - covers dispatch, weighing, packing, documentation and handover

- No source held for the outturn reconciliation half of this task

- No source held. HM_PP_009 Product Quality Management System folder arrived empty.

- KBK-MIR-LAB-PRE-SOP-009 Pengiriman Sampel ke laboratorium eksternal

- 4034-PR-PRO-002 Sampling Protocol

- RA-01 Resin Loss Register and RA-02 Carbon Contamination Register

# 10  REVIEW

This procedure and the documents beneath it are reviewed every 12 months, and immediately after any incident, plant change, equipment change or change to the schedule.

# 11  REGISTER TRACEABILITY

The three-digit suffix is shared between the two registers, so SWI-PRO-MET-202-011 is the work instruction for JSEA-PRO-MET-202-011.

Registers: HM-MMM-WHS-REG-JSEA-PRO-updated_10.xlsx tab JSEA_MET, and HM-MMM-WHS-REG-XXX-R00-SWI-SWMS.xlsx tab SWI-MET. GGT working masters are MM-4034-033-OR-03-JSEA-Register-RevE.xlsx and MM-4034-033-OR-03-SWI-Register-RevE.xlsx.

| Item | Count |
| --- | --- |
| Register lines in area 202 | 12 |
| Hazard analyses issued | 12 |
| Work instructions issued | 0 |
| Desk-based tasks with a JSEA but no work instruction | 12 |
| Safety-critical | 2 |
| Issued but NOT FOR USE | 1 |