Safe Work Procedure

Laboratory Instrumental Analysis

SWP-214

| DOCUMENT HISTORY AND STATUS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Document: | Laboratory Instrumental Analysis Safe Work Procedure |  |  | Version No.: | A |  |  |
| Document No: | SWP-214 |  |  | Owner: | Process Operations Superintendent |  |  |
| Issue Date: | 11/08/2026 |  |  | Date Reviewed: | - |  |  |
| History: | Process owner: Heritage Mineral Operations, Business unit Mt. Morgan. Document approver: Process Manager. |  |  |  |  |  |  |
| Date Reviewed: | Version No.: | Written by: |  | Reviewed by: |  | Authorised by: |  |
| 11/08/2026 | A | GGT (cost code 4034-033) |  |  |  |  |  |
| Revision Description: |  |  |  |  |  |  |  |
| Changes: |  |  |  |  |  |  |  |
| Rev A - first issue. Converted from the GGT Standard Work Procedure layout to the Heritage Safe Work Procedure format; content unchanged. |  |  |  |  |  |  |  |
| Action: | Name: |  | Position: |  |  |  | Date: |
| Prepared by | GGT (cost code 4034-033) |  | Document preparer |  |  |  | 11/08/2026 |
| Reviewed by |  |  | Process Operations Superintendent |  |  |  |  |
| Reviewed by |  |  |  |  |  |  |  |
| Authorised by |  |  | Process Manager |  |  |  |  |

This document is controlled electronically as part of Heritage Minerals Pty Limited’s Health & Safety Management System.  Please refer to Heritage Minerals Controlled Document Register for the current version.

Table of Contents

1.	Purpose and Scope	3

2.	Responsibilities	3

3.	Abbreviations and Definitions	3

4.	Potential Hazards	4

5.	Personal Protective Equipment	4

6.	Procedure	5

6.1	Introduction	5

6.2	Procedure and Implementation Steps	5

6.3	Schedule	5

6.4	Flowchart	6

7.	Training	6

8.	Document Traceability	6

9.	Review Criteria	7

10.	Legislation & References	7

10.1	Legislation	7

10.2	Source Documents by Task	7

# Purpose and Scope

This procedure covers laboratory instrumental analysis at the Mt. Morgan processing plant.

Laboratory instrumental analysis - atomic absorption spectrometry start-up, reading and calibration, burner and nebuliser maintenance, gas cylinder handling, the carbon and sulphur analyser, instrument fault response, and result entry and release.

Document details:

Hazard analyses beneath: 14 (JSEA-PRO-LAB-303-001 to -014)

Work instructions beneath: 13

Process owner: Heritage Mineral Operations, Business unit Mt. Morgan

Document owner: Process Operations Superintendent

Document approver: Process Manager

# Responsibilities

| Staff | Responsibilities |
| --- | --- |
| Laboratory Technician | - Carry out the task to the schedule and to the work instruction. - Only carry out work where competent and authorised to do so. - Record what was actually done. Work not done is recorded as not done - never invented and never back-filled. - Stop and report if any control in the JSEA is not in place. |
| Laboratory Supervisor | - Authorise the task and confirm the area is fit before it starts. - Know that the task is in progress and where the person is. - Understand that much of the plant is automated and equipment may start or stop without warning. - Act on any off-specification result reported from the task. |
| Chief Chemist / Metallurgist | - Set and maintain the schedule - what is done, how often, and what is reported. - Review the results and act on the trends. - Investigate any variance outside the agreed tolerance before a period is closed. |
| Laboratory Manager / Superintendent | - Ensure all personnel are trained and competent. - Ensure this procedure and the documents beneath it are reviewed at the stated interval. - Approve the risk ranking in the JSEAs. |

# Abbreviations and Definitions

| Term | Meaning |
| --- | --- |
| JSEA | Job safety and environmental analysis - one per task, giving the step-by-step hazards, controls and risk ranking. |
| SWI | Safe work instruction - one per task, giving the step-by-step method. |
| SWP | Safe work procedure - this document. |
| Steady state | Normal operation held for at least 15 to 20 minutes. |
| ReCYN | The resin-in-pulp cyanide and metal recovery process in Areas 041, 051, 071 and 081. |
| CN Free / WAD CN | Free cyanide by silver nitrate titration; weak acid dissociable cyanide by distillation. |

# Potential Hazards

The step-by-step hazard analysis for each task, with the risk ranking, is in the JSEA for that task. The hazards common across this area are:

Contact with live electrical equipment at a sample pump, cutter or local control station

Inhalation of acid fume or reagent vapour at the bench

Release from a pressurised sample line, hose or fitting

Contact with hot solution or hot surface at the elution, stripping or regeneration circuits

Cuts from broken glassware, and burns from hot glassware

Cross-contamination from a container or cutter not cleaned between points

Non-representative sample leading to a wrong process decision

Manual handling injury from carrying sample buckets, containers and composite drums

An accounting or reconciliation error that hides a real metal loss

Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt

# Personal Protective Equipment

Standard PPE is worn for all normal operations: hard hat, safety glasses, high-visibility long-sleeved shirt and long trousers, safety boots, and gloves appropriate to the task.

The following is added where the task requires it:

Personal HCN gas monitor, calibrated and in test date

Chemical splash goggles and face shield

Nitrile rubber gloves

Nitrile PVC boots

Chemical resistant suit

Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

Acid-resistant gloves, apron and face shield

Chemical splash goggles

# Procedure

## Introduction

| Warning: | many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times. |
| --- | --- |

## Procedure and Implementation Steps

The documents below are the controlled method for each task. This procedure does not repeat them.

| JSEA | Work instruction | Task | Class |
| --- | --- | --- | --- |
| JSEA-PRO-LAB-303-001 | SWI-PRO-LAB-303-001 | AAS Start-Up, Flame Ignition and Shutdown | Safety-critical |
| JSEA-PRO-LAB-303-002 | SWI-PRO-LAB-303-002 | AAS Sample Reading - Gold and Silver | Safety-critical |
| JSEA-PRO-LAB-303-003 | SWI-PRO-LAB-303-003 | AAS Sample Reading - Base Metals | Safety-critical |
| JSEA-PRO-LAB-303-004 | SWI-PRO-LAB-303-004 | AAS Calibration, Standardisation and Drift Check | Safety-critical |
| JSEA-PRO-LAB-303-005 | SWI-PRO-LAB-303-005 | Acetylene and Air Cylinder Changeover and Leak Test | Safety-critical |
| JSEA-PRO-LAB-303-006 | SWI-PRO-LAB-303-006 | AAS Burner and Nebuliser Cleaning and Maintenance | Safety-critical |
| JSEA-PRO-LAB-303-007 | SWI-PRO-LAB-303-007 | Spectrophotometer (HACH DR) Operation and Verification | Operating |
| JSEA-PRO-LAB-303-008 | SWI-PRO-LAB-303-008 | Instrument Fault Response and Sample Re-Run Protocol | Operating |
| JSEA-PRO-LAB-303-009 | - | Result Entry, Verification and Release to Plant | Operating |
| JSEA-PRO-LAB-303-010 | SWI-PRO-LAB-303-010 | Carbon and Sulphur Analyser Operation and Sample Loading - Eltra CS-i | Safety-critical |
| JSEA-PRO-LAB-303-011 | SWI-PRO-LAB-303-011 | Carbon and Sulphur Analyser Calibration, Standards and Drift Check | Safety-critical |
| JSEA-PRO-LAB-303-012 | SWI-PRO-LAB-303-012 | Carbon and Sulphur Analyser Crucible, Accelerator and Furnace Cleaning | Safety-critical |
| JSEA-PRO-LAB-303-013 | SWI-PRO-LAB-303-013 | Oxygen and Argon Supply, Regulator and Flashback Arrestor Check | Safety-critical |
| JSEA-PRO-LAB-303-014 | SWI-PRO-LAB-303-014 | AAS Extraction Blower, Vent and UPS Function Check | Safety-critical |

## Schedule

| Task | Frequency |
| --- | --- |
| 303-001 AAS Start-Up, Flame Ignition and Shutdown | Every analytical session. |
| 303-002 AAS Sample Reading - Gold and Silver | Every gold or silver batch. |
| 303-003 AAS Sample Reading - Base Metals | Every base metal batch. |
| 303-004 AAS Calibration, Standardisation and Drift Check | Every session, and at the drift check interval within it. |
| 303-005 Acetylene and Air Cylinder Changeover and Leak Test | Every cylinder change. |
| 303-006 AAS Burner and Nebuliser Cleaning and Maintenance | At the interval the method sets and whenever sensitivity falls. |
| 303-007 Spectrophotometer (HACH DR) Operation and Verification | To be set once an instrument is purchased. |
| 303-008 Instrument Fault Response and Sample Re-Run Protocol | Every instrument fault. |
| 303-009 Result Entry, Verification and Release to Plant | Every batch. |
| 303-010 Carbon and Sulphur Analyser Operation and Sample Loading - Eltra CS-i | To be set once the method and consumables are resolved. |
| 303-011 Carbon and Sulphur Analyser Calibration, Standards and Drift Check | To be set once standards are purchased. |
| 303-012 Carbon and Sulphur Analyser Crucible, Accelerator and Furnace Cleaning | To be set once the instrument is commissioned. |
| 303-013 Oxygen and Argon Supply, Regulator and Flashback Arrestor Check | At the set interval and at every cylinder change. |
| 303-014 AAS Extraction Blower, Vent and UPS Function Check | Every session, and monthly in full. |

## Flowchart

Not reproduced here. Each work instruction carries its own step sequence.

# Training

Every person carrying out a task in this area is trained and signed off against the work instruction for that task, and has signed on to the matching JSEA at a communication session.

Site induction, current.

Cyanide awareness and gas detection competency where the area requires it.

Working at height competency where the task requires it.

Competency in the determinations, instruments and calculations used in the task.

Refresher at the interval set by the site training matrix, and after any change to the instruction.

# Document Traceability

The three-digit suffix is shared between the hazard analysis and the work instruction, so SWI-PRO-LAB-303-011 is the work instruction for JSEA-PRO-LAB-303-011.

| Item | Count |
| --- | --- |
| Tasks in area 303 | 14 |
| Hazard analyses issued | 14 |
| Work instructions issued | 13 |
| Desk-based tasks with a JSEA but no work instruction | 1 |
| Safety-critical | 11 |
| Issued but NOT FOR USE | 4 |

# Review Criteria

This Procedure and the documents beneath it shall be reviewed as follows:

Every 12 months; or

Immediately after any incident, plant change, equipment change or change to the schedule.

# Legislation & References

## Legislation

Queensland Mining and Quarrying Safety and Health Act 1999

Queensland Mining and Quarrying Safety and Health Regulation 2017

## Source Documents by Task

The source documents and notes held for each task.

| JSEA | Task | Source documents and notes |
| --- | --- | --- |
| JSEA-PRO-LAB-303-001 | AAS Start-Up, Flame Ignition and Shutdown | - ILS quote Q0023979: 2 x PerkinElmer PinAAcle 500 Syngistix. KBK SOPs are written for GBC instruments and will need rewriting, not adapting |
| JSEA-PRO-LAB-303-002 | AAS Sample Reading - Gold and Silver | - ILS quote Q0023979: PinAAcle 500 with air-acetylene burner and high-sensitivity nebulisers |
| JSEA-PRO-LAB-303-003 | AAS Sample Reading - Base Metals | - ILS quote Q0023979: PinAAcle 500, flame only - no graphite furnace or ICP quoted |
| JSEA-PRO-LAB-303-004 | AAS Calibration, Standardisation and Drift Check | - ILS quote Q0023979: PinAAcle 500 Syngistix; PerkinElmer method and calibration guidance not held |
| JSEA-PRO-LAB-303-005 | Acetylene and Air Cylinder Changeover and Leak Test | - KBK-MIR-LAB-AAS-SOP-066 Penggantian tabung gas asetilen |
| JSEA-PRO-LAB-303-006 | AAS Burner and Nebuliser Cleaning and Maintenance | - ILS quote Q0023979 includes high-sensitivity nebulisers and o-ring kits but no maintenance procedure is held |
| JSEA-PRO-LAB-303-007 | Spectrophotometer (HACH DR) Operation and Verification | - No spectrophotometer of any kind in ILS quote Q0023979. The HACH DR2800 methods cannot be used. Add an instrument or delete this line and every colorimetric method that depends on it |
| JSEA-PRO-LAB-303-008 | Instrument Fault Response and Sample Re-Run Protocol | - No source document held |
| JSEA-PRO-LAB-303-009 | Result Entry, Verification and Release to Plant | - No LIMS or result release procedure held - depends on the lab systems decision |
| JSEA-PRO-LAB-303-010 | Carbon and Sulphur Analyser Operation and Sample Loading - Eltra CS-i | - ILS quote Q0023979: Eltra CS-i carbon and sulphur analyser, 2 sulphur channels 0.0001-4.6 per cent. No vendor manual held |
| JSEA-PRO-LAB-303-011 | Carbon and Sulphur Analyser Calibration, Standards and Drift Check | - ILS quote Q0023979 states plainly that no standards or consumables are included with the Eltra CS-i |
| JSEA-PRO-LAB-303-012 | Carbon and Sulphur Analyser Crucible, Accelerator and Furnace Cleaning | - No crucible, accelerator or furnace cleaning procedure held for the Eltra CS-i |
| JSEA-PRO-LAB-303-013 | Oxygen and Argon Supply, Regulator and Flashback Arrestor Check | - ILS quote Q0023979: 2-stage pure oxygen/argon regulator, acetylene regulators and flashback arrestors; KBK AAS-SOP-066 is the pattern |
| JSEA-PRO-LAB-303-014 | AAS Extraction Blower, Vent and UPS Function Check | - ILS quote Q0023979: 2 x Riello UPS 2200VA and 2 x blower and vent assemblies for the AAS. No check procedure held |