Safe Work Procedure

Laboratory Sample Preparation

HM-PRC-VXX-PRO212

| DOCUMENT HISTORY AND STATUS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Document: | Laboratory Sample Preparation Safe Work Procedure |  |  | Version No.: | A |  |  |
| Document No: | HM-PRC-VXX-PRO212 |  |  | Owner: | Process Operations Superintendent |  |  |
| Issue Date: | 11/08/2026 |  |  | Date Reviewed: | - |  |  |
| History: | Process owner: Heritage Mineral Operations, Business unit Mt. Morgan. Document approver: Process Manager. |  |  |  |  |  |  |
| Date Reviewed: | Version No.: | Written by: |  | Reviewed by: |  | Authorised by: |  |
| 11/08/2026 | A | GGT (cost code 4034-033) |  |  |  |  |  |
| Revision Description: |  |  |  |  |  |  |  |
| Changes: |  |  |  |  |  |  |  |
| Rev A - first issue. Converted from the GGT Standard Work Procedure layout to the Heritage Safe Work Procedure format; content unchanged. |  |  |  |  |  |  |  |
| Action: | Name: |  | Position: |  |  |  | Date: |
| Prepared by | GGT (cost code 4034-033) |  | Document preparer |  |  |  | 11/08/2026 |
| Reviewed by |  |  | Process Operations Superintendent |  |  |  |  |
| Reviewed by |  |  |  |  |  |  |  |
| Authorised by |  |  | Process Manager |  |  |  |  |

This document is controlled electronically as part of Heritage Minerals Pty Limited’s Health & Safety Management System.  Please refer to Heritage Minerals Controlled Document Register for the current version.

Table of Contents

1.	Purpose and Scope	3

2.	Responsibilities	3

3.	Abbreviations and Definitions	4

4.	Potential Hazards	4

5.	Personal Protective Equipment	4

6.	Procedure	5

6.1	Introduction	5

6.2	Procedure and Implementation Steps	5

6.3	Schedule	6

6.4	Flowchart	6

7.	Training	6

8.	Register Traceability	7

9.	Review Criteria	7

10.	Legislation & References	7

10.1	Legislation	7

10.2	Source Documents by Task	7

# Purpose and Scope

This procedure covers laboratory sample preparation at the Mt. Morgan processing plant. It is the procedure that the 17 hazard analyses and 17 work instructions in register area 301 sit under.

Laboratory sample preparation - receipt and registration, sorting and batching, drying, splitting, pulverising, fineness checking, packaging and storage, dispatch, residue handling, and the dust collector, compressor and balances that serve the area.

Document details:

Register area: 301 Sample Preparation, on tabs JSEA_LAB and SWI-LAB

Hazard analyses beneath: 17 (JSEA-PRO-LAB-301-001 to -017)

Work instructions beneath: 17

Process owner: Heritage Mineral Operations, Business unit Mt. Morgan

Document owner: Process Operations Superintendent

Document approver: Process Manager

# Responsibilities

| Staff | Responsibilities |
| --- | --- |
| Laboratory Technician | - Carry out the task to the schedule and to the work instruction. - Only carry out work where competent and authorised to do so. - Record what was actually done. Work not done is recorded as not done - never invented and never back-filled. - Stop and report if any control in the JSEA is not in place. |
| Laboratory Supervisor | - Authorise the task and confirm the area is fit before it starts. - Know that the task is in progress and where the person is. - Understand that much of the plant is automated and equipment may start or stop without warning. - Act on any off-specification result reported from the task. |
| Chief Chemist / Metallurgist | - Set and maintain the schedule - what is done, how often, and what is reported. - Review the results and act on the trends. - Investigate any variance outside the agreed tolerance before a period is closed. |
| Laboratory Manager / Superintendent | - Ensure all personnel are trained and competent. - Ensure this procedure and the documents beneath it are reviewed at the stated interval. - Approve the risk ranking in the JSEAs. |

# Abbreviations and Definitions

| Term | Meaning |
| --- | --- |
| JSEA | Job safety and environmental analysis - one per task, giving the step-by-step hazards, controls and risk ranking. |
| SWI | Safe work instruction - one per task, giving the step-by-step method. |
| SWP | Safe work procedure - this document. |
| Steady state | Normal operation held for at least 15 to 20 minutes. |
| ReCYN | The resin-in-pulp cyanide and metal recovery process in Areas 041, 051, 071 and 081. |
| CN Free / WAD CN | Free cyanide by silver nitrate titration; weak acid dissociable cyanide by distillation. |

# Potential Hazards

The step-by-step hazard analysis for each task, with the risk ranking, is in the JSEA for that task. The hazards common across this area are:

Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt

Hearing damage in the mill, screen, compressor and blower areas

Contact with an agitator, sample cutter, pump or screen that starts without warning

Manual handling injury from carrying sample buckets, containers and composite drums

Cross-contamination from a container or cutter not cleaned between points

Non-representative sample leading to a wrong process decision

Contact with hot solution or hot surface at the elution, stripping or regeneration circuits

Inhalation of acid fume or reagent vapour at the bench

Cuts from broken glassware, and burns from hot glassware

Contact with live electrical equipment at a sample pump, cutter or local control station

Release from a pressurised sample line, hose or fitting

Hydrogen cyanide gas released at the open sample point

Slip, trip or fall on wet or slurry-covered walkways around sample points

# Personal Protective Equipment

Standard PPE is worn for all normal operations: hard hat, safety glasses, high-visibility long-sleeved shirt and long trousers, safety boots, and gloves appropriate to the task.

The following is added where the task requires it:

Personal HCN gas monitor, calibrated and in test date

Chemical splash goggles and face shield

Nitrile rubber gloves

Nitrile PVC boots

Chemical resistant suit

Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

Acid-resistant gloves, apron and face shield

Chemical splash goggles

# Procedure

## Introduction

| Warning: | many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times. |
| --- | --- |

## Procedure and Implementation Steps

The documents below are the controlled method for each task. This procedure does not repeat them.

| JSEA | Work instruction | Task | Class |
| --- | --- | --- | --- |
| JSEA-PRO-LAB-301-001 | SWI-PRO-LAB-301-001 | Sample Receipt, Registration and Chain of Custody | Operating |
| JSEA-PRO-LAB-301-002 | SWI-PRO-LAB-301-002 | Sample Sorting and Batch Allocation | Operating |
| JSEA-PRO-LAB-301-003 | SWI-PRO-LAB-301-003 | Sample Drying - Drying Oven Operation | Safety-critical |
| JSEA-PRO-LAB-301-004 | SWI-PRO-LAB-301-004 | Jaw Crusher Operation | Operating |
| JSEA-PRO-LAB-301-005 | SWI-PRO-LAB-301-005 | Jaw Crusher Liner Change and Blockage Clearing | Operating |
| JSEA-PRO-LAB-301-006 | SWI-PRO-LAB-301-006 | Sample Splitting Before Pulverising - Riffle and Rotary Splitter | Operating |
| JSEA-PRO-LAB-301-007 | SWI-PRO-LAB-301-007 | Pulverising - Ring Mill Operation | Operating |
| JSEA-PRO-LAB-301-008 | SWI-PRO-LAB-301-008 | Pulveriser Bowl Cleaning and Cross-Contamination Control | Safety-critical |
| JSEA-PRO-LAB-301-009 | SWI-PRO-LAB-301-009 | Sample Fineness (Pulverising Quality) Check | Operating |
| JSEA-PRO-LAB-301-010 | SWI-PRO-LAB-301-010 | Pulp Sample Packaging, Labelling and Storage | Operating |
| JSEA-PRO-LAB-301-011 | SWI-PRO-LAB-301-011 | Sample Dispatch to External and Umpire Laboratory | Operating |
| JSEA-PRO-LAB-301-012 | SWI-PRO-LAB-301-012 | Residue and Reject Pulp Handling and Disposal | Safety-critical |
| JSEA-PRO-LAB-301-013 | SWI-PRO-LAB-301-013 | Dust Collector Operation and Cleaning - Prep Area | Safety-critical |
| JSEA-PRO-LAB-301-014 | SWI-PRO-LAB-301-014 | Compressor Condensate Drain and Service - Prep Area | Safety-critical |
| JSEA-PRO-LAB-301-015 | SWI-PRO-LAB-301-015 | Manual Handling, Trolley and Light Vehicle Use in Prep Area | Operating |
| JSEA-PRO-LAB-301-016 | SWI-PRO-LAB-301-016 | Analytical Balance Verification - Prep Area | Operating |
| JSEA-PRO-LAB-301-017 | SWI-PRO-LAB-301-017 | Rotary Sample Divider Operation and Bucket Set Change - RSD 010L | Operating |

## Schedule

| Task | Frequency |
| --- | --- |
| 301-001 Sample Receipt, Registration and Chain of Custody | Every delivery from the plant or the field. |
| 301-002 Sample Sorting and Batch Allocation | Every registered batch. |
| 301-003 Sample Drying - Drying Oven Operation | Every wet sample before preparation. |
| 301-004 Jaw Crusher Operation | To be set once the crushing scope is resolved. |
| 301-005 Jaw Crusher Liner Change and Blockage Clearing | To be set once the crushing scope is resolved. |
| 301-006 Sample Splitting Before Pulverising - Riffle and Rotary Splitter | Every sample before pulverising. |
| 301-007 Pulverising - Ring Mill Operation | Every sample. |
| 301-008 Pulveriser Bowl Cleaning and Cross-Contamination Control | Between every sample. |
| 301-009 Sample Fineness (Pulverising Quality) Check | At the frequency the quality procedure sets, and after any pulveriser service. |
| 301-010 Pulp Sample Packaging, Labelling and Storage | Every prepared pulp. |
| 301-011 Sample Dispatch to External and Umpire Laboratory | Every dispatch. |
| 301-012 Residue and Reject Pulp Handling and Disposal | Continuous, with disposal at the retention interval. |
| 301-013 Dust Collector Operation and Cleaning - Prep Area | Daily check; cleaning at the set interval. |
| 301-014 Compressor Condensate Drain and Service - Prep Area | Daily drain; service at the manufacturer's interval. |
| 301-015 Manual Handling, Trolley and Light Vehicle Use in Prep Area | Every day. |
| 301-016 Analytical Balance Verification - Prep Area | Daily verification before use; external calibration at the set interval. |
| 301-017 Rotary Sample Divider Operation and Bucket Set Change - RSD 010L | Every sample split on the divider. |

## Flowchart

Not reproduced here. Each work instruction carries its own step sequence.

# Training

Every person carrying out a task in this area is trained and signed off against the work instruction for that task, and has signed on to the matching JSEA at a communication session.

Site induction, current.

Cyanide awareness and gas detection competency where the area requires it.

Working at height competency where the task requires it.

Competency in the determinations, instruments and calculations used in the task.

Refresher at the interval set by the site training matrix, and after any change to the instruction.

# Register Traceability

The three-digit suffix is shared between the two registers, so SWI-PRO-LAB-301-011 is the work instruction for JSEA-PRO-LAB-301-011.

Registers: HM-MMM-WHS-REG-JSEA-PRO-updated_10.xlsx tab JSEA_LAB, and HM-MMM-WHS-REG-XXX-R00-SWI-SWMS.xlsx tab SWI-LAB. GGT working masters are MM-4034-033-OR-03-JSEA-Register-RevF.xlsx and MM-4034-033-OR-03-SWI-Register-RevF.xlsx.

| Item | Count |
| --- | --- |
| Register lines in area 301 | 17 |
| Hazard analyses issued | 17 |
| Work instructions issued | 17 |
| Desk-based tasks with a JSEA but no work instruction | 0 |
| Safety-critical | 5 |
| Issued but NOT FOR USE | 2 |

# Review Criteria

This Procedure and the documents beneath it shall be reviewed as follows:

Every 12 months; or

Immediately after any incident, plant change, equipment change or change to the schedule.

# Legislation & References

## Legislation

Queensland Mining and Quarrying Safety and Health Act 1999

Queensland Mining and Quarrying Safety and Health Regulation 2017

## Source Documents by Task

The source documents and notes held for each task. Register lines are on tabs JSEA_LAB and SWI-LAB (Section 8).

| JSEA | Task | Source documents and notes |
| --- | --- | --- |
| JSEA-PRO-LAB-301-001 | Sample Receipt, Registration and Chain of Custody | - KBK-MIR-LAB-PRE-SOP-001 Penerimaan Sampel |
| JSEA-PRO-LAB-301-002 | Sample Sorting and Batch Allocation | - KBK-MIR-LAB-PRE-SOP-002 Sortir Sampel |
| JSEA-PRO-LAB-301-003 | Sample Drying - Drying Oven Operation | - KBK-MIR-LAB-PRE-SOP-003; ILS quote Q0023979: 2 x 560 L drying ovens and 70 stainless trays |
| JSEA-PRO-LAB-301-004 | Jaw Crusher Operation | - No jaw crusher or hammer mill anywhere in ILS quote Q0023979. Confirm the laboratory receives no rock samples - if so this line and 301-005 should be deleted |
| JSEA-PRO-LAB-301-005 | Jaw Crusher Liner Change and Blockage Clearing | - No crusher purchased, so KBK PRE-SOP-105 and -110 do not apply. Delete with 301-004 once confirmed |
| JSEA-PRO-LAB-301-006 | Sample Splitting Before Pulverising - Riffle and Rotary Splitter | - KBK-MIR-LAB-PRE-SOP-006 Splitting before pulverising |
| JSEA-PRO-LAB-301-007 | Pulverising - Ring Mill Operation | - KBK-MIR-LAB-PRE-SOP-007; ILS quote Q0023979: 2 x PV2 pulverising mills 380/415 V with bowl sets |
| JSEA-PRO-LAB-301-008 | Pulveriser Bowl Cleaning and Cross-Contamination Control | - ILS quote Q0023979: spacer rings B800/B1000 and B125/B300/B400 quoted, but no contamination-control procedure held |
| JSEA-PRO-LAB-301-009 | Sample Fineness (Pulverising Quality) Check | - KBK-MIR-LAB-PRE-SOP-008 Uji Kehalusan Sampel |
| JSEA-PRO-LAB-301-010 | Pulp Sample Packaging, Labelling and Storage | - KBK-MIR-LAB-WET-SOP-048 Pengemasan Sampel Pulp |
| JSEA-PRO-LAB-301-011 | Sample Dispatch to External and Umpire Laboratory | - KBK-MIR-LAB-PRE-SOP-009 Pengiriman sampel ke laboratorium eksternal |
| JSEA-PRO-LAB-301-012 | Residue and Reject Pulp Handling and Disposal | - KBK-MIR-LAB-PRE-SOP-010 Pemindahan residu dan pulp sampel |
| JSEA-PRO-LAB-301-013 | Dust Collector Operation and Cleaning - Prep Area | - KBK-MIR-LAB-PRE-SOP-107; ILS quote Q0023979: XLC 2-8 4000 CFM c/w VSD. Pulse-system wiring, isolator, GPO and air connections are excluded from ILS scope |
| JSEA-PRO-LAB-301-014 | Compressor Condensate Drain and Service - Prep Area | - KBK-MIR-LAB-PRE-SOP-011; ILS quote Q0023979: Pneumark PM 7.5 kW compressor with dryer and 600 L tank |
| JSEA-PRO-LAB-301-015 | Manual Handling, Trolley and Light Vehicle Use in Prep Area | - KBK-MIR-LAB-PRE-SOP-109 and WET-SOP-053 manual handling and LV |
| JSEA-PRO-LAB-301-016 | Analytical Balance Verification - Prep Area | - KBK-MIR-LAB-PRE-SOP-012; ILS quote Q0023979: Balance FG-150KAM 150 kg x 0.05 kg |
| JSEA-PRO-LAB-301-017 | Rotary Sample Divider Operation and Bucket Set Change - RSD 010L | - ILS quote Q0023979: RSD 010L rotary sample divider and 25 per cent bucket set; KBK PRE-SOP-006 is the nearest method |