Safe Work Procedure

Laboratory Services, Safety and Quality

HM-PRC-VXX-PRO216

| DOCUMENT HISTORY AND STATUS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Document: | Laboratory Services, Safety and Quality Safe Work Procedure |  |  | Version No.: | A |  |  |
| Document No: | HM-PRC-VXX-PRO216 |  |  | Owner: | Process Operations Superintendent |  |  |
| Issue Date: | 11/08/2026 |  |  | Date Reviewed: | - |  |  |
| History: | Process owner: Heritage Mineral Operations, Business unit Mt. Morgan. Document approver: Process Manager. |  |  |  |  |  |  |
| Date Reviewed: | Version No.: | Written by: |  | Reviewed by: |  | Authorised by: |  |
| 11/08/2026 | A | GGT (cost code 4034-033) |  |  |  |  |  |
| Revision Description: |  |  |  |  |  |  |  |
| Changes: |  |  |  |  |  |  |  |
| Rev A - first issue. Converted from the GGT Standard Work Procedure layout to the Heritage Safe Work Procedure format; content unchanged. |  |  |  |  |  |  |  |
| Action: | Name: |  | Position: |  |  |  | Date: |
| Prepared by | GGT (cost code 4034-033) |  | Document preparer |  |  |  | 11/08/2026 |
| Reviewed by |  |  | Process Operations Superintendent |  |  |  |  |
| Reviewed by |  |  |  |  |  |  |  |
| Authorised by |  |  | Process Manager |  |  |  |  |

This document is controlled electronically as part of Heritage Minerals Pty Limited’s Health & Safety Management System.  Please refer to Heritage Minerals Controlled Document Register for the current version.

Table of Contents

1.	Purpose and Scope	3

2.	Responsibilities	3

3.	Abbreviations and Definitions	4

4.	Potential Hazards	4

5.	Personal Protective Equipment	4

6.	Procedure	5

6.1	Introduction	5

6.2	Procedure and Implementation Steps	5

6.3	Schedule	6

6.4	Flowchart	7

7.	Training	7

8.	Register Traceability	7

9.	Review Criteria	7

10.	Legislation & References	8

10.1	Legislation	8

10.2	Source Documents by Task	8

# Purpose and Scope

This procedure covers laboratory services, safety and quality at the Mt. Morgan processing plant. It is the procedure that the 16 hazard analyses and 12 work instructions in register area 305 sit under.

Laboratory services, safety and quality - quality control and control charts, umpire assay management, chemical store and dangerous goods, cyanide handling, gas alarm response, emergency showers, ventilation and scrubber, waste, housekeeping, gold security, the maintenance and calibration register, compressed air and the pressure vessel, emergency equipment, and equipment commissioning.

Document details:

Register area: 305 Laboratory Services, on tabs JSEA_LAB and SWI-LAB

Hazard analyses beneath: 16 (JSEA-PRO-LAB-305-001 to -016)

Work instructions beneath: 12

Process owner: Heritage Mineral Operations, Business unit Mt. Morgan

Document owner: Process Operations Superintendent

Document approver: Process Manager

# Responsibilities

| Staff | Responsibilities |
| --- | --- |
| Laboratory Technician | - Carry out the task to the schedule and to the work instruction. - Only carry out work where competent and authorised to do so. - Record what was actually done. Work not done is recorded as not done - never invented and never back-filled. - Stop and report if any control in the JSEA is not in place. |
| Laboratory Supervisor | - Authorise the task and confirm the area is fit before it starts. - Know that the task is in progress and where the person is. - Understand that much of the plant is automated and equipment may start or stop without warning. - Act on any off-specification result reported from the task. |
| Chief Chemist / Metallurgist | - Set and maintain the schedule - what is done, how often, and what is reported. - Review the results and act on the trends. - Investigate any variance outside the agreed tolerance before a period is closed. |
| Laboratory Manager / Superintendent | - Ensure all personnel are trained and competent. - Ensure this procedure and the documents beneath it are reviewed at the stated interval. - Approve the risk ranking in the JSEAs. |

# Abbreviations and Definitions

| Term | Meaning |
| --- | --- |
| JSEA | Job safety and environmental analysis - one per task, giving the step-by-step hazards, controls and risk ranking. |
| SWI | Safe work instruction - one per task, giving the step-by-step method. |
| SWP | Safe work procedure - this document. |
| Steady state | Normal operation held for at least 15 to 20 minutes. |
| ReCYN | The resin-in-pulp cyanide and metal recovery process in Areas 041, 051, 071 and 081. |
| CN Free / WAD CN | Free cyanide by silver nitrate titration; weak acid dissociable cyanide by distillation. |

# Potential Hazards

The step-by-step hazard analysis for each task, with the risk ranking, is in the JSEA for that task. The hazards common across this area are:

Cross-contamination from a container or cutter not cleaned between points

Non-representative sample leading to a wrong process decision

An accounting or reconciliation error that hides a real metal loss

Exposure to a gold security incident, and to suspicion in one

Contact with sulphuric or hydrochloric acid - burn to skin or eyes

Contact with sodium hydroxide solution - burn to skin or eyes

Hydrogen cyanide gas released at the open sample point

Manual handling injury from carrying sample buckets, containers and composite drums

Inhalation of acid fume or reagent vapour at the bench

Cuts from broken glassware, and burns from hot glassware

Splash of cyanide-bearing slurry or solution to skin or eyes

Slip, trip or fall on wet or slurry-covered walkways around sample points

Contact with live electrical equipment at a sample pump, cutter or local control station

Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt

# Personal Protective Equipment

Standard PPE is worn for all normal operations: hard hat, safety glasses, high-visibility long-sleeved shirt and long trousers, safety boots, and gloves appropriate to the task.

The following is added where the task requires it:

Personal HCN gas monitor, calibrated and in test date

Chemical splash goggles and face shield

Nitrile rubber gloves

Nitrile PVC boots

Chemical resistant suit

Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

Acid-resistant gloves, apron and face shield

Chemical splash goggles

# Procedure

## Introduction

| Warning: | many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times. |
| --- | --- |

## Procedure and Implementation Steps

The documents below are the controlled method for each task. This procedure does not repeat them.

| JSEA | Work instruction | Task | Class |
| --- | --- | --- | --- |
| JSEA-PRO-LAB-305-001 | - | Laboratory QA/QC - Standards, Blanks, Duplicates and Repeats | Operating |
| JSEA-PRO-LAB-305-002 | - | Control Chart Review and Out-of-Control Response | Operating |
| JSEA-PRO-LAB-305-003 | - | Umpire and Referee Assay Management | Safety-critical |
| JSEA-PRO-LAB-305-004 | SWI-PRO-LAB-305-004 | Laboratory Chemical Store Management and SDS Control | Safety-critical |
| JSEA-PRO-LAB-305-005 | SWI-PRO-LAB-305-005 | Cyanide Handling and Storage in the Laboratory | Safety-critical |
| JSEA-PRO-LAB-305-006 | SWI-PRO-LAB-305-006 | HCN Gas Alarm Response - Laboratory | Safety-critical |
| JSEA-PRO-LAB-305-007 | SWI-PRO-LAB-305-007 | Emergency Shower and Eyewash Inspection and Test | Operating |
| JSEA-PRO-LAB-305-008 | SWI-PRO-LAB-305-008 | Laboratory Ventilation and Scrubber System Check | Safety-critical |
| JSEA-PRO-LAB-305-009 | SWI-PRO-LAB-305-009 | Laboratory Waste Segregation, Storage and Disposal | Safety-critical |
| JSEA-PRO-LAB-305-010 | SWI-PRO-LAB-305-010 | Laboratory Housekeeping and Deep Clean | Safety-critical |
| JSEA-PRO-LAB-305-011 | SWI-PRO-LAB-305-011 | Gold Security and Sample Integrity Control | Safety-critical |
| JSEA-PRO-LAB-305-012 | - | Laboratory Equipment Preventive Maintenance and Calibration Register | Operating |
| JSEA-PRO-LAB-305-013 | SWI-PRO-LAB-305-013 | Dangerous Goods Cabinet Management - Flammable, Corrosive and Toxic/Poisons | Safety-critical |
| JSEA-PRO-LAB-305-014 | SWI-PRO-LAB-305-014 | Laboratory Compressed Air System and Pressure Vessel Statutory Inspection | Safety-critical |
| JSEA-PRO-LAB-305-015 | SWI-PRO-LAB-305-015 | Spill Kit, Fire Extinguisher and Emergency Equipment Inspection | Safety-critical |
| JSEA-PRO-LAB-305-016 | SWI-PRO-LAB-305-016 | Laboratory Equipment Receipt, Installation and Commissioning Handover | Safety-critical |

## Schedule

| Task | Frequency |
| --- | --- |
| 305-001 Laboratory QA/QC - Standards, Blanks, Duplicates and Repeats | Every analytical batch. |
| 305-002 Control Chart Review and Out-of-Control Response | Weekly review; response on every out-of-control event. |
| 305-003 Umpire and Referee Assay Management | Every umpire submission and every settlement dispute. |
| 305-004 Laboratory Chemical Store Management and SDS Control | Continuous; register reviewed at the set interval. |
| 305-005 Cyanide Handling and Storage in the Laboratory | Every cyanide handling task. |
| 305-006 HCN Gas Alarm Response - Laboratory | On every alarm; drill at the set interval. |
| 305-007 Emergency Shower and Eyewash Inspection and Test | Weekly test; annual full inspection. |
| 305-008 Laboratory Ventilation and Scrubber System Check | Every shift; full check at the set interval. |
| 305-009 Laboratory Waste Segregation, Storage and Disposal | Continuous; disposal at the set interval. |
| 305-010 Laboratory Housekeeping and Deep Clean | Daily; deep clean at the set interval. |
| 305-011 Gold Security and Sample Integrity Control | Continuous. |
| 305-012 Laboratory Equipment Preventive Maintenance and Calibration Register | Continuous; reviewed monthly. |
| 305-013 Dangerous Goods Cabinet Management - Flammable, Corrosive and Toxic/Poisons | Continuous; inspected at the set interval. |
| 305-014 Laboratory Compressed Air System and Pressure Vessel Statutory Inspection | Daily drain; statutory inspection at the regulated interval. |
| 305-015 Spill Kit, Fire Extinguisher and Emergency Equipment Inspection | Monthly inspection; annual service. |
| 305-016 Laboratory Equipment Receipt, Installation and Commissioning Handover | Once at laboratory establishment; then on every new instrument. |

## Flowchart

Not reproduced here. Each work instruction carries its own step sequence.

# Training

Every person carrying out a task in this area is trained and signed off against the work instruction for that task, and has signed on to the matching JSEA at a communication session.

Site induction, current.

Cyanide awareness and gas detection competency where the area requires it.

Working at height competency where the task requires it.

Competency in the determinations, instruments and calculations used in the task.

Refresher at the interval set by the site training matrix, and after any change to the instruction.

# Register Traceability

The three-digit suffix is shared between the two registers, so SWI-PRO-LAB-305-011 is the work instruction for JSEA-PRO-LAB-305-011.

Registers: HM-MMM-WHS-REG-JSEA-PRO-updated_10.xlsx tab JSEA_LAB, and HM-MMM-WHS-REG-XXX-R00-SWI-SWMS.xlsx tab SWI-LAB. GGT working masters are MM-4034-033-OR-03-JSEA-Register-RevF.xlsx and MM-4034-033-OR-03-SWI-Register-RevF.xlsx.

| Item | Count |
| --- | --- |
| Register lines in area 305 | 16 |
| Hazard analyses issued | 16 |
| Work instructions issued | 12 |
| Desk-based tasks with a JSEA but no work instruction | 4 |
| Safety-critical | 12 |
| Issued but NOT FOR USE | 0 |

# Review Criteria

This Procedure and the documents beneath it shall be reviewed as follows:

Every 12 months; or

Immediately after any incident, plant change, equipment change or change to the schedule.

# Legislation & References

## Legislation

Queensland Mining and Quarrying Safety and Health Act 1999

Queensland Mining and Quarrying Safety and Health Regulation 2017

## Source Documents by Task

The source documents and notes held for each task. Register lines are on tabs JSEA_LAB and SWI-LAB (Section 8).

| JSEA | Task | Source documents and notes |
| --- | --- | --- |
| JSEA-PRO-LAB-305-001 | Laboratory QA/QC - Standards, Blanks, Duplicates and Repeats | - No QA/QC procedure in the KBK set - a gap in their system as well as ours |
| JSEA-PRO-LAB-305-002 | Control Chart Review and Out-of-Control Response | - No source document held |
| JSEA-PRO-LAB-305-003 | Umpire and Referee Assay Management | - KBK-MIR-LAB-PRE-SOP-009 covers dispatch only, not umpire assay management |
| JSEA-PRO-LAB-305-004 | Laboratory Chemical Store Management and SDS Control | - KBK-MIR-LAB-WET-SOP-025 Pengambilan bahan kimia |
| JSEA-PRO-LAB-305-005 | Cyanide Handling and Storage in the Laboratory | - KBK-MIR-LAB-WET-SOP-025; ILS quote Q0023979: 60 L toxic and poisons cabinet |
| JSEA-PRO-LAB-305-006 | HCN Gas Alarm Response - Laboratory | - SWI-022 Gas Detector Alarm Response (HCN and H2S), issued |
| JSEA-PRO-LAB-305-007 | Emergency Shower and Eyewash Inspection and Test | - ILS quote Q0023979: safety shower with eye wash plus 3 separate eye wash stations |
| JSEA-PRO-LAB-305-008 | Laboratory Ventilation and Scrubber System Check | - KBK WET-SOP-054 and ENV-SOP-061; ILS quote Q0023979: Model A crossflow scrubber, exhaust fan, pH control and 12 m of 400 mm ducting |
| JSEA-PRO-LAB-305-009 | Laboratory Waste Segregation, Storage and Disposal | - KBK-MIR-LAB-ENV-SOP-058 and -106 waste handling and neutralisation |
| JSEA-PRO-LAB-305-010 | Laboratory Housekeeping and Deep Clean | - KBK-MIR-LAB-WET-SOP-059, ENV-SOP-060 and -062 cleaning procedures |
| JSEA-PRO-LAB-305-011 | Gold Security and Sample Integrity Control | - ILS quote Q0023979: Safe NV1 single shelf in the bullion assay area |
| JSEA-PRO-LAB-305-012 | Laboratory Equipment Preventive Maintenance and Calibration Register | - ILS quote Q0023979 is the asset list the register should be built from |
| JSEA-PRO-LAB-305-013 | Dangerous Goods Cabinet Management - Flammable, Corrosive and Toxic/Poisons | - ILS quote Q0023979: 160 L flammable, 160 L corrosive and 60 L toxic/poisons cabinets; KBK WET-SOP-025 |
| JSEA-PRO-LAB-305-014 | Laboratory Compressed Air System and Pressure Vessel Statutory Inspection | - ILS quote Q0023979: Pneumark PM 7.5 kW, 43 cfm, 600 L receiver. A registered pressure vessel in Queensland - statutory inspection applies |
| JSEA-PRO-LAB-305-015 | Spill Kit, Fire Extinguisher and Emergency Equipment Inspection | - ILS quote Q0023979: 120 L hazchem spill kit, 3 x 9 kg extinguishers, first aid kit. No inspection routine held |
| JSEA-PRO-LAB-305-016 | Laboratory Equipment Receipt, Installation and Commissioning Handover | - ILS quote Q0023979 is the asset list and the commissioning milestone; lead time 12 to 18 weeks ex works |