Safe Work Instruction

Reagent Solution Strength Sampling - NaCN, NaOH, H2SO4, NaCl, Lime and NaSH

This document is the property of Heritage Minerals with all rights are reserved in respect of it. This document cannot be reproduced without the written consent of Heritage Minerals

# Purpose and Scope

This Safe Work Instruction is required so that the mechanical and operational integrity of the system is kept intact. It assists operators and mechanical technicians alike to identify possible problems before they happen.  It will help ensure that when personnel are performing this task it is performed in a safe and secure manner, to ensure the operational integrity of the machine and applies when this task is performed either by itself or in conjunction with other tasks in the area.

A JSA or Take 5 must be completed and discussed with the Supervisor if any variation to this SWI is necessary.

# RESPONSIBILITIES AND AUTHORITY

| What | Who | When |
| --- | --- | --- |
| Conform to procedure: Implement Changes: Change procedure: Review: Authorisation: | All personnel Anyone Training Facilitator Working party of content experts Manager | At all times As required As required On change & yearly On change |

# PPE REQUIREMENTS

- Safety helmet and safety glasses

- Chemical splash goggles and face shield

- High-visibility long-sleeved shirt and long trousers

- Safety boots with sound tread - nitrile PVC boots at the sample point

- Chemical resistant suit, acid-resistant and nitrile rubber gloves, and apron

- Personal HCN gas monitor and personal multi-gas monitor including H2S, calibrated and in test date

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

In the event of an emergency

Call on CH19 “EMERGENCY, EMERGENCY, EMERGENCY” on the project to make everyone aware, call 000 for emergency services, police, fire or ambulance, you will need to stay calm and state when requested by a responder:

• Your Name. • Your Location. • Type of Emergency; and • Assistance Required.

| Part 1: Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: |  |  | SWI-PRO-MET-201-020 - Reagent Solution Strength Sampling - NaCN, NaOH, H2SO4, NaCl, Lime and NaSH |  |  |  |  |  |  |  |  |  |  |  |
| Issued Date: |  |  | 11/08/2026 |  |  |  |  | Next Review Date: |  |  |  | 11/08/2027 |  |  |
| Document Owner: |  |  | Processing |  |  |  |  | JSEA ID: |  |  |  | JSEA-PRO-MET-201-020 |  |  |
| Part 2: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of work: |  |  |  |  |  | Confirm every made-up reagent is at the strength the process assumes before it is dosed. A reagent made up off-strength is dosed at the wrong rate everywhere it is used, and the error is not visible until the circuit drifts. Sample lime slaker feed, product and final product, bagged hydrated lime, ReCYN plant caustic, and the made-up flocculant solution (Steps 4 to 7). Area: 101 to 107 Reagents. Points and equipment covered: 101 - Sodium cyanide, solution and briquettes 102 - Sodium hydroxide, diluted 103 - Lime, slaked slurry 104 - Sodium chloride, made-up solution 105 - Sulphuric acid, diluted 107 - Sodium hydrosulphide, as delivered and dosed Flocculant mixing area - made-up flocculant solution (moved here from the tails thickener SWI) Frequency: Every batch made up, and once per shift on the standing solutions. Draft for review: Steps 4 to 6 are taken from the Martabe work instructions Sampling Feed, Product and Final Product of Lime Slaker (DOC-3-MET-PRS-WIN-00059-IE), Sampling Hydrated Lime (DOC-3-MET-PRS-WIN-00061-IE) and ReCYN Plant Caustic Strength Measurement (DOC-3-MET-PMC-WIN-00088-IE). Step 7 is taken from Tailing Flocculant Solution Sampling (DOC-3-MET-PRS-WIN-00064-IE), moved here from the area of the tails thickener. All v1.0, 25/12/2024. JSEA-PRO-MET-201-020 must be updated for the added hazards before this instruction is approved. Open items: (1) map the Martabe lime slaker feed hopper, product hopper and lime tank to the Transmin slaking plant points; (2) the source gives no test for the lime samples, no slurry temperature and no bag type; (3) confirm the test on hydrated lime - per cent solids on dry lime is unusual (lime availability is in SWI-PRO-MET-205-005); (4) confirm the ReCYN caustic pump drain pipe carries the diluted solution; (5) the flocculant solution strength test is not stated; (6) confirm the record folders and forms exist at Mt. Morgan. This is a safety-critical task. It is not carried out alone in a cyanide area, and not without the Shift Supervisor knowing it is in progress. |  |  |  |  |  |  |  |  |
| Potential Hazards: |  |  |  |  |  | Hydrogen cyanide gas released at the open sample point — Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Contact with sulphuric or hydrochloric acid - burn to skin or eyes — Full acid PPE including face shield, acid-resistant gloves, apron and boots. Sample drawn only from the designated sample valve. Add acid to water, never the reverse. Safety shower and eyewash within reach and proven flowing. Spill kit at the point. Contact with sodium hydroxide solution - burn to skin or eyes — Full chemical PPE including face shield and chemical-resistant gloves. Sample drawn slowly from the designated valve. Safety shower and eyewash within reach and proven flowing. Hydrogen sulphide released from sodium hydrosulphide or from an acidified sulphide stream — Personal multi-gas monitor including H2S worn and in calibration date. Area ventilation confirmed running. Two people, one clear. Withdraw upwind on any alarm. No sampling of the NaSH circuit during acid dosing. Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt — Sampling of solid cyanide done only with the full cyanide PPE set including a gas mask with an A2B2E2K2P3 cartridge. Dust suppression and local extraction confirmed running. No dry sweeping. Splash of cyanide-bearing slurry or solution to skin or eyes — Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Slip, trip or fall on wet or slurry-covered walkways around sample points — Spillage cleaned up immediately and the point left clean. Walkways kept clear of hoses, buckets and containers. Safety boots with sound tread. Report any damaged or missing grating before use. Manual handling injury from carrying sample buckets, containers and composite drums — Sample volume kept to the schedule - no over-filling. Two-person lift or a trolley for composite containers above the site manual handling limit. Route cleared before lifting. Working alone in a cyanide area where a collapse would not be seen — Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. Non-representative sample leading to a wrong process decision — Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. Cross-contamination from a container or cutter not cleaned between points — Bucket, container, cutter and sieve rinsed with process water and then with the stream being sampled, and the rinse discarded, before every sample. Dedicated containers for high-grade streams. Labels checked against the schedule before handover. [JSEA to confirm] Lime dust and contact when sampling bagged hydrated lime, and heat and dust at the lime slaker — Hazmat overall, latex gloves, N95 mask, glasses, hard hat and safety boots. Eyewash proven flowing before starting; wash dust from the eyes at once; wash skin with soap under running water. Lime safety data sheet read. Sampler position watched. Forklift moves done only by an authorised operator. No water in the lime feed pocket at the slaker. [JSEA to confirm] Spilled caustic and sulphuric acid, and contact with the fumes, at the ReCYN caustic sample point and bench — Nitrile gloves, goggles and safety boots. Carry reagents in closed bottles or glassware. Safety data sheets read; eyewash proven flowing; wash contact under running water and soap. Titrate only in a working fume cupboard; close the sample bottle tightly after sampling. [JSEA to confirm] Flocculant solution contact, slippery floor and pinch point at the flocculant mixing area — Latex or chemical-resistant gloves, hard hat, glasses and safety boots. Open and close valves with the body in an ergonomic position, without hurrying; stop at any pain. Hands clear of the feed valve pinch points. Watch footing on wet floors. No sampling during flocculant lifting. The full step-by-step hazard analysis, with the risk ranking, is in JSEA-PRO-MET-201-020 Section 4. |  |  |  |  |  |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT (PPE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part 3: Safe Work Instruction Steps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Job Number (Hazards/Cautions) |  | Application Method |  |  |  |  |  |  | Visual Reference |  |  |  |  |  |
| - Pre-start Check - Working alone in a cyanide area - Gas monitor or fixed detection not healthy - Safety shower or eyewash not proven flowing - Wrong reagent procedure or PPE |  | - Confirm you are trained and signed off against this SWI, signed on to JSEA-PRO-MET-201-020 at a communication session, site inducted with current cyanide awareness and gas detection competency where the area requires it, and authorised by the Shift Supervisor. - Confirm a second person is in the area. Cyanide-area sampling is not done alone. - Read the reagent-specific site sampling procedure before starting - the PPE and the approach differ between cyanide, acid, caustic and NaSH. - NEVER use a container that has held a different reagent. Dedicated, labelled containers only. - Confirm the plant is in steady-state operation - normal operation for a minimum of 15 to 20 minutes before sampling. A sample taken during a swing is not representative. - Notify the control room or DCS operator that the sampling round is starting, and which points it covers. - Confirm the personal gas monitor is on, in calibration date and reading clean, and that fixed detection in the area is healthy. - Confirm the safety shower and eyewash nearest the point are within reach and proven flowing. - Label every container BEFORE sampling - point, date, time and shift. Equipment Required : - Sample bottles 250 mL, one dedicated set per reagent - pH meter - Hydrometer or density meter - Titration set - burette, pipette, conical flask, stand, wash bottle - Standardised AgNO3 for cyanide; standardised HCl for caustic; standardised NaOH for acid - Rhodanine and phenolphthalein indicators - Distilled water - Sample containers (sampler) kept at the lime slaker, dry plastic bags, scoop, marker, cable ties, forklift (hydrated lime bags), N95 mask, hazmat overall - 100 mL sample bottle with a closing lid, fume cupboard, sulphuric acid solution prepared per the Martabe instruction (not received) - Small sample bottle for the flocculant solution - Personal protective equipment as listed in PPE Requirements, matched to the reagent being sampled |  |  |  |  |  |  |  |  |  |  |  |  |
| - Made-up Reagent Strength Sampling - HCN released at the cyanide sample point - Acid or caustic burn to skin or eyes - H2S released from sodium hydrosulphide - Dust from cyanide briquettes |  | - For sodium cyanide solution, sample only from the designated valve with full cyanide PPE, including the gas mask with A2B2E2K2P3 cartridge available for immediate use. - For sodium cyanide briquettes, sample the solid with the dust controls in place and do not open the sample in an unventilated space. - For sulphuric acid and sodium hydroxide, sample slowly from the designated diluted-solution valve, never from the concentrated storage tank. - For sodium hydrosulphide, confirm no acid dosing is running anywhere that could contact the stream, and wear the multi-gas monitor including H2S. CAUTION: Add acid to water, never the reverse. Never sample from a concentrated storage tank. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Field Measurement and Records - Batch outside target dosed - Strength not recorded against the sample |  | - Titrate or measure density in the field where the method allows, and bottle the laboratory split for confirmation. - Record batch number, make-up time, measured strength and the target strength against each sample. Report any batch outside the target range to the Shift Supervisor before it is dosed. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Lime Slaker Feed, Product and Final Product Sampling - Lime dust and contact with the product - Heat from the lime slurry - Sampler striking the head - Wet bag or damaged bag |  | - Take 5 minutes before starting (Take 5). Check the plastic bags are not damaged or leaking. - Feed: the sampling point is the feed hopper. Use the sampler kept near the lime slaker. Put the sampler into the feed bin and fill it. Take it out of the bin. Make sure the bag is dry (change a wet bag for a dry one). Put the sample in the bag and tie it tightly. - Product: the sampling point is the product hopper. Open the product bin cover, put the sampler in and fill it, lift it out. Use a dry bag, put the sample in and tie it tightly. Close the product bin cover. - Final product: the sampling point is the lime tank. Put the sampler into the final product bin, fill it, lift it out. Use a dry bag, put the sample in and tie it tightly. CAUTION: Hazards are dust, contact with the product, the sampler hitting the head, and heat from the lime slurry. Hard hat on. Make sure there is no water in the lime feed pocket. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Bagged Hydrated Lime Sampling - Lime dust to the eyes or skin - Forklift operated by an unauthorised person - Sample not homogeneous - Sample bag leaking |  | - Check the nearest eyewash works. Prepare plastic bags, marker and scoop. Have the lime safety data sheet available. - Bring the sample bag down with the forklift as the forklift operation instruction requires. Only an authorised forklift operator does this. - Write the ID number on the plastic bag. Untie the sample bag and level the surface of the opened lime bag so lime does not spill. - Take the sample with a scoop, spread evenly so the sample is homogeneous across the bag. Take it at least 10 cm below the surface. - Put the sample in the labelled plastic bag. Twist the bag tightly and tie it by hand so no air is left inside. Also tie it with a cable tie. Double-bag it and tie the outer bag the same way. - Close the sample container again. Send the sample and the dispatch form to the Mt. Morgan laboratory. - Record the per cent solids of the hydrated lime in G:\Processing\5. Metallurgy\Metallurgy Lab\04 Metlab Data\Lime\% Solid Hydrated Lime. |  |  |  |  |  |  |  |  |  |  |  |  |
| - ReCYN Plant Caustic - Sampling and Strength - Caustic or sulphuric acid spill and fumes - Sample bottle overfilled - Wrong titration result |  | - Take 5 and prepare the fume cupboard, beakers, 100 mL sample bottle, dropper and burette. Check them and the reagents (water, NaOH, phenolphthalein, sulphuric acid) are good to use. - Take the NaOH sample from the drain pipe of the ReCYN plant caustic pump into the 100 mL bottle. Fill the bottle to the level of the lid so it does not overflow, then close it tightly. - Prepare the sulphuric acid and titrate the sample in the fume cupboard as SWI-PRO-MET-205-002 Part B describes. - Record the caustic strength in G:\Processing\5. Metallurgy\Metallurgy Lab\04 Metlab Data\Concentrasi NaOH or on form DOC-4-MET-PMC-DFR-00135-EN (Form Caustic Strength.xlsx). CAUTION: Sample only the diluted solution, never the concentrated tank. Carry reagents in closed bottles or glassware. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Flocculant Solution Sampling - Flocculant solution contact - Slippery floor at the flocculant mixing area - Pinch point at the feed valve - Sampling during flocculant lifting or handling |  | - Take 5 minutes before starting (Take 5). Confirm the controls in Part 2 are in place and the sample bottle is ready and in good condition. If not, stop and report. - Do not sample while flocculant lifting or handling is under way - wait until it has finished. - The sampling point is the flocculant tube in the flocculant mixing area. Open one of the control valves next to the sampling tube, on the line with the pump running. - Open the tube cover and wait for the tube to fill with flocculant solution. Close the valve when the tube is full. - Put the bottle into the tube and fill it to the top with flocculant solution. - To empty the tube, close the feed valve and the control valve together, then return the valves to their normal position. Close the tube cover. CAUTION: Slippery floors and pinch points at the feed valve are the main hazards. Keep hands clear of the feed valve and wear gloves, hard hat, glasses and safety boots. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Completion and Clean Up - Sample valve left weeping - Samples not handed over with a signed record - Control room not told the round is complete |  | - Close the sample valve or stop the sample pump and confirm the point is not weeping. - Wash down the point and the surrounding walkway. Leave nothing on the grating. - Record sampling time, operator name and the operating conditions read at the time - flow, density and any instrument value available. - Deliver the labelled samples to the laboratory or to the composite station as the schedule requires, and sign the handover. - Notify the control room that the round is complete. - Record the result, the method used and the instrument or balance identity on the field or bench data sheet - date, time, shift, operator and every value determined. Make the register or logbook entry and any handover signature. - Record any step not completed, with the reason. - Report to the Shift Supervisor, and record, any control in Part 2 found not in place. |  |  |  |  |  |  |  |  |  |  |  |  |

| Referenced Documents |  |
| --- | --- |
| Document ID | Document Title |
| JSEA-PRO-MET-201-020 | JSEA — Reagent Solution Strength Sampling - NaCN, NaOH, H2SO4, NaCl, Lime and NaSH |
| HM-PRC-V01-PRO021 to PRO027 | Reagent circuit procedures |
| Mt. Morgan site procedures 19, 20, 21 and 22 | Sodium Cyanide Solution, Sodium Cyanide Briquettes, Diluted Sulphuric Acid and Diluted Sodium Hydroxide Sampling [LIVE] |
| KBK-MIR-MP-PRO-MET-SOP-0009 | Caustic soda titration |
| KBK-MIR-MP-PRO-MET-SOP-0010 | Cyanide titration |
| KBK-MIR-MP-PRO-MET-SOP-0025, 0039 and 0040 | Hydrochloric acid titration, received 11 Aug 2026 |
| KBK-MIR-MP-PRO-MET-SOP-0041 and 0042 | Sulphuric acid titration, received 11 Aug 2026 |
| KBK-MIR-MP-PRO-MET-SOP-0043 | AgNO3 standard preparation, received 11 Aug 2026 |
| KBK-MIR-MP-PRO-OPE-SOP-0076 | Fasilitas Cyanide, received 11 Aug 2026 |
| 4034-046-VE-019 Rev A | Transmin quicklime slaking plant functional description, received 11 Aug 2026 |
| SWI-PRO-MET-205-002 | Caustic Soda Strength Titration (Part B) |
| SWI-PRO-MET-205-005 | Lime availability |
| SWI-PRO-MET-204-010 | Flocculant Screening and Dose Optimisation Test |
| DOC-3-MET-PRS-WIN-00059-IE v1.0 | Martabe WI Sampling Feed, Product and Final Product of Lime Slaker (25/12/2024) - source of Step 4 |
| DOC-3-MET-PRS-WIN-00061-IE v1.0 | Martabe WI Sampling Hydrated Lime (25/12/2024) - source of Step 5 |
| DOC-3-MET-PMC-WIN-00088-IE v1.0 | Martabe WI ReCYN Plant Caustic Strength Measurement (25/12/2024) - source of Step 6 |
| DOC-3-MET-PRS-WIN-00064-IE v1.0 | Martabe WI Tailing Flocculant Solution Sampling (25/12/2024) - source of Step 7 |
| DOC-2-MET-PRM-SOP-00037-IE | Martabe SOP referenced by the Martabe WIs - not received |

In the event of an emergency

1. HCN Gas Alarm If the personal monitor or fixed detection alarms (5 ppm alarm, 10 ppm high-high), stop work, leave the area upwind and call CH19 “EMERGENCY”. Do not re-enter until the Shift Supervisor clears the area. 2. Hydrogen Sulphide Smell or Alarm If H2S is smelled or the multi-gas monitor alarms, stop work, leave the area upwind and call CH19 “EMERGENCY”. Do not re-enter until the Shift Supervisor clears the area. 3. Suspected Cyanide Exposure If a person becomes dizzy, short of breath or collapses, call CH19 “EMERGENCY”, call 000, and bring the cyanide antidote kit and a trained first aider. Do not give mouth-to-mouth resuscitation. 4. Skin or Eye Contact with Reagent Flush with copious water at the safety shower or eyewash for a minimum of 15 minutes. Remove contaminated clothing while flushing. Report to the first aider. 5. HCl Gas or Acid Contact Leave the area upwind. Flush skin or eyes with copious water at the safety shower or eyewash for a minimum of 15 minutes. If breathing is affected call CH19 “EMERGENCY” and call 000. 6. Slip, Fall or Lifting Injury Call CH19 “EMERGENCY”, call 000 if the injury is serious, and notify the Shift Supervisor.