Safe Work Instruction

Sampling Under Cyanide Alarm or Restricted Access Conditions

This document is the property of Heritage Minerals with all rights are reserved in respect of it. This document cannot be reproduced without the written consent of Heritage Minerals

# Purpose and Scope

This Safe Work Instruction is required so that the mechanical and operational integrity of the system is kept intact. It assists operators and mechanical technicians alike to identify possible problems before they happen.  It will help ensure that when personnel are performing this task it is performed in a safe and secure manner, to ensure the operational integrity of the machine and applies when this task is performed either by itself or in conjunction with other tasks in the area.

A JSA or Take 5 must be completed and discussed with the Supervisor if any variation to this SWI is necessary.

# RESPONSIBILITIES AND AUTHORITY

| What | Who | When |
| --- | --- | --- |
| Conform to procedure: Implement Changes: Change procedure: Review: Authorisation: | All personnel Anyone Training Facilitator Working party of content experts Manager | At all times As required As required On change & yearly On change |

# PPE REQUIREMENTS

- Safety helmet and safety glasses

- Chemical splash goggles and face shield

- High-visibility long-sleeved shirt and long trousers

- Safety boots with sound tread - nitrile PVC boots at the sample point

- Chemical resistant suit, nitrile rubber gloves and apron

- Personal HCN gas monitor and personal multi-gas monitor including H2S, calibrated and in test date

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

In the event of an emergency

Call on CH19 “EMERGENCY, EMERGENCY, EMERGENCY” on the project to make everyone aware, call 000 for emergency services, police, fire or ambulance, you will need to stay calm and state when requested by a responder:

• Your Name. • Your Location. • Type of Emergency; and • Assistance Required.

| Part 1: Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: |  |  | SWI-PRO-MET-201-025 - Sampling Under Cyanide Alarm or Restricted Access Conditions |  |  |  |  |  |  |  |  |  |  |  |
| Issued Date: |  |  | 11/08/2026 |  |  |  |  | Next Review Date: |  |  |  | 11/08/2027 |  |  |
| Document Owner: |  |  | Processing |  |  |  |  | JSEA ID: |  |  |  | JSEA-PRO-MET-201-025 |  |  |
| Part 2: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of work: |  |  |  |  |  | Set what happens to the sampling round when a gas alarm sounds or access is restricted. The short answer is that sampling stops. This instruction says how it stops, and how it restarts. Area: 201 Plant Sampling. Points and equipment covered: All cyanide-area sample points - under gas alarm or restricted access Frequency: Whenever a gas alarm sounds or access is restricted. This instruction sits under Standard Work Procedure HM-PRC-VXX-PRO205 - Plant Sampling and Metallurgical Data Collection. Read that procedure before carrying out this task for the first time. Register line: SWI-PRO-MET-201-025 in HM-MMM-WHS-REG-XXX-R00-SWI-SWMS, tab SWI-MET. The matching hazard analysis is JSEA-PRO-MET-201-025. This is a safety-critical task. It is not carried out alone in a cyanide area, and not without the Shift Supervisor knowing it is in progress. |  |  |  |  |  |  |  |  |
| Potential Hazards: |  |  |  |  |  | Hydrogen cyanide gas released at the open sample point — Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Hydrogen sulphide released from sodium hydrosulphide or from an acidified sulphide stream — Personal multi-gas monitor including H2S worn and in calibration date. Area ventilation confirmed running. Two people, one clear. Withdraw upwind on any alarm. No sampling of the NaSH circuit during acid dosing. Working alone in a cyanide area where a collapse would not be seen — Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. Slip, trip or fall on wet or slurry-covered walkways around sample points — Spillage cleaned up immediately and the point left clean. Walkways kept clear of hoses, buckets and containers. Safety boots with sound tread. Report any damaged or missing grating before use. Non-representative sample leading to a wrong process decision — Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. The full step-by-step hazard analysis, with the risk ranking, is in JSEA-PRO-MET-201-025 Section 4. |  |  |  |  |  |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT (PPE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part 3: Safe Work Instruction Steps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Job Number (Hazards/Cautions) |  | Application Method |  |  |  |  |  |  | Visual Reference |  |  |  |  |  |
| - Pre-start Check - Working alone in a cyanide area - Monitor not carried or not healthy - No radio contact |  | - Confirm you are trained and signed off against this SWI, signed on to JSEA-PRO-MET-201-025 at a communication session, site inducted with current cyanide awareness and gas detection competency where the area requires it, and authorised by the Shift Supervisor. - Confirm a second person is in the area. Cyanide-area sampling is not done alone. - Confirm the personal HCN monitor and, where H2S is credible, the multi-gas monitor are carried, calibrated and switched on. - Confirm you carry a radio. - Notify the control room or DCS operator that the sampling round is starting, and which points it covers. - Confirm the personal gas monitor is on, in calibration date and reading clean, and that fixed detection in the area is healthy. - Confirm the safety shower and eyewash nearest the point are within reach and proven flowing. - Label every container BEFORE sampling - point, date, time and shift. Equipment Required : - Personal HCN gas monitor - Personal multi-gas monitor where H2S is credible - Radio - Personal protective equipment as listed in PPE Requirements |  |  |  |  |  |  |  |  |  |  |  |  |
| - On a Personal Monitor Alarm - HCN or H2S exposure - Returning for the sample or equipment |  | - On any personal monitor alarm: stop, hold your breath, move upwind and uphill, and do not return to collect the sample or the equipment. CAUTION: Do not return to collect the sample or the equipment. |  |  |  |  |  |  |  |  |  |  |  |  |
| - On a Fixed Detection Alarm or Siren - Delay leaving the area - Alarm not reported |  | - On any fixed detection alarm or area siren: leave the area by the nearest safe route to the muster point. The site has 20 gas detectors set at 5 ppm alarm and 10 ppm high-high. - Report the alarm and your location to the control room as soon as you are clear. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Account for Others and Do Not Re-enter - Person left in the area - Re-entering to close a valve - Area entered before it is clear |  | - Account for anyone else who was in the area with you. Sampling in cyanide areas is never done alone, so there is always someone to account for. - Do NOT re-enter to close a sample valve you have left open. Tell the control room it is open and let the response deal with it. - Do not return to the area until the Shift Supervisor confirms it is clear and the cause is understood. CAUTION: Do not re-enter until the Shift Supervisor confirms the area is clear. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Missed Samples and Restart - Missed samples not recorded - Open sample point not inspected |  | - Record the missed samples as missed, with the alarm time and the restriction period. - Once the area is released, inspect every sample point you had opened before restarting the round. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Restricted Access - Finding a way round the restriction |  | - Where access is restricted for another reason - a lift, a confined space entry, hot work - the affected points are not sampled and are recorded as restricted. Do not find a way around the restriction to get the sample. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Completion - Control room not told the round stopped - Register not completed |  | - Notify the control room that the round is complete, or that it is stopped and why. - Record any step not completed, with the reason. - Report to the Shift Supervisor, and record, any control in Part 2 found not in place. - Record every value determined on the field or bench data sheet - date, time, shift and operator. Make the register or logbook entry and any handover signature. |  |  |  |  |  |  |  |  |  |  |  |  |

| Referenced Documents |  |
| --- | --- |
| Document ID | Document Title |
| HM-PRC-VXX-PRO205 | Plant Sampling and Metallurgical Data Collection |
| JSEA-PRO-MET-201-025 | JSEA — Sampling Under Cyanide Alarm or Restricted Access Conditions |
| HM-MMM-WHS-REG-XXX-R00-SWI-SWMS | SWI / SWMS Register — tab SWI-MET |
| SWI-022 | Gas Detector Alarm Response (HCN and H2S), issued Rev A |
| Control Philosophy 3.1.x | 20 gas detectors at 5 ppm alarm and 10 ppm high-high |
| KBK-MIR-MP-PRO-OPE-SOP-0076 | Fasilitas Cyanide, received 11 Aug 2026 |

In the event of an emergency

1. HCN Gas Alarm If the personal monitor or fixed detection alarms (5 ppm alarm, 10 ppm high-high), stop work, leave the area upwind and call CH19 “EMERGENCY”. Do not re-enter until the Shift Supervisor clears the area. 2. Hydrogen Sulphide Smell or Alarm If H2S is smelled or the multi-gas monitor alarms, stop work, leave the area upwind and call CH19 “EMERGENCY”. Do not re-enter until the Shift Supervisor clears the area. 3. Suspected Cyanide Exposure If a person becomes dizzy, short of breath or collapses, call CH19 “EMERGENCY”, call 000, and bring the cyanide antidote kit and a trained first aider. Do not give mouth-to-mouth resuscitation. 4. Slip, Fall or Lifting Injury Call CH19 “EMERGENCY”, call 000 if the injury is serious, and notify the Shift Supervisor.