Safe Work Instruction

CIL Sampling Round - 032-TK-002 to 032-TK-006 Launders

This document is the property of Heritage Minerals with all rights are reserved in respect of it. This document cannot be reproduced without the written consent of Heritage Minerals

# Purpose and Scope

This Safe Work Instruction is required so that the mechanical and operational integrity of the system is kept intact. It assists operators and mechanical technicians alike to identify possible problems before they happen.  It will help ensure that when personnel are performing this task it is performed in a safe and secure manner, to ensure the operational integrity of the machine and applies when this task is performed either by itself or in conjunction with other tasks in the area.

A JSA or Take 5 must be completed and discussed with the Supervisor if any variation to this SWI is necessary.

# RESPONSIBILITIES AND AUTHORITY

| What | Who | When |
| --- | --- | --- |
| Conform to procedure: Implement Changes: Change procedure: Review: Authorisation: | All personnel Anyone Training Facilitator Working party of content experts Manager | At all times As required As required On change & yearly On change |

# PPE REQUIREMENTS

- Safety helmet and safety glasses

- Chemical splash goggles and face shield

- High-visibility long-sleeved shirt and long trousers

- Safety boots with sound tread - nitrile PVC boots at the sample point

- Chemical resistant suit, nitrile rubber gloves and apron

- Personal HCN gas monitor, calibrated and in test date

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Harness and fall arrest where no fixed platform exists

In the event of an emergency

Call on CH19 “EMERGENCY, EMERGENCY, EMERGENCY” on the project to make everyone aware, call 000 for emergency services, police, fire or ambulance, you will need to stay calm and state when requested by a responder:

• Your Name. • Your Location. • Type of Emergency; and • Assistance Required.

| Part 1: Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: |  |  | SWI-PRO-MET-201-004 - CIL Sampling Round - 032-TK-002 to 032-TK-006 Launders |  |  |  |  |  |  |  |  |  |  |  |
| Issued Date: |  |  | 11/08/2026 |  |  |  |  | Next Review Date: |  |  |  | 11/08/2027 |  |  |
| Document Owner: |  |  | Processing |  |  |  |  | JSEA ID: |  |  |  | JSEA-PRO-MET-201-004 |  |  |
| Part 2: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of work: |  |  |  |  |  | Set the gold profile down the CIL train, tank by tank. Set pH, free and WAD cyanide and dissolved oxygen in each stage. Give the metallurgist the stage-by-stage solution profile that shows whether adsorption is working and where it is falling over. Check the carbon carried over into the launder, by sampling at the launder. This shows carbon loss down the train. Area: 032 Carbon in Leach. Points and equipment covered: 032-TK-002 - CIL tank 1 launder 032-TK-003 - CIL tank 2 launder 032-TK-004 - CIL tank 3 launder 032-TK-005 - CIL tank 4 launder 032-TK-006 - CIL tank 5 launder, final CIL tank Frequency: Once per shift across all five tanks; CIL-1 and CIL-5 hourly. Carbon-in-launder check (Step 5): hourly, as a routine check. Draft for review: the carbon-in-launder check (Step 5) is taken from the Martabe work instruction Sampling Carbon Launder CIL (DOC-3-MET-PRS-WIN-00058-IE v1.0, 25/12/2024). The sampler volume, oven temperature, drying time and concentration calculation are not stated in the source and must be confirmed. JSEA-PRO-MET-201-004 must be updated for the added hazard (sampler striking the head, pinch point at the sampling point cover) before this instruction is approved. This is a safety-critical task. It is not carried out alone in a cyanide area, and not without the Shift Supervisor knowing it is in progress. |  |  |  |  |  |  |  |  |
| Potential Hazards: |  |  |  |  |  | Hydrogen cyanide gas released at the open sample point — Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Splash of cyanide-bearing slurry or solution to skin or eyes — Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Fall from a tank top, launder platform, column platform or access ladder — Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. Contact with an agitator, sample cutter, pump or screen that starts without warning — Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. Slip, trip or fall on wet or slurry-covered walkways around sample points — Spillage cleaned up immediately and the point left clean. Walkways kept clear of hoses, buckets and containers. Safety boots with sound tread. Report any damaged or missing grating before use. Manual handling injury from carrying sample buckets, containers and composite drums — Sample volume kept to the schedule - no over-filling. Two-person lift or a trolley for composite containers above the site manual handling limit. Route cleared before lifting. Working alone in a cyanide area where a collapse would not be seen — Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. Non-representative sample leading to a wrong process decision — Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. Cross-contamination from a container or cutter not cleaned between points — Bucket, container, cutter and sieve rinsed with process water and then with the stream being sampled, and the rinse discarded, before every sample. Dedicated containers for high-grade streams. Labels checked against the schedule before handover. Confined space atmosphere at a tank top hatch, launder or hopper opening — No entry. Sampling is done from outside the opening only. Confined space entry, if it is ever needed for a sample point, requires a permit, atmosphere testing, isolation, a standby person and a rescue plan under the site confined space procedure. [JSEA to confirm] The sampler striking the head, and a pinch point at the sampling point cover — Hard hat on and the position of the sampler watched at all times. Open and close the cover slowly with hands clear of the pinch point and a safe distance kept. Slurry poured slowly. Glasses, latex gloves and safety boots; HCN controls as above. The full step-by-step hazard analysis, with the risk ranking, is in JSEA-PRO-MET-201-004 Section 4. |  |  |  |  |  |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT (PPE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part 3: Safe Work Instruction Steps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Job Number (Hazards/Cautions) |  | Application Method |  |  |  |  |  |  | Visual Reference |  |  |  |  |  |
| - Pre-start Check - Working alone in a cyanide area - Gas monitor or fixed detection not healthy - Safety shower or eyewash not proven flowing - Sampler or strainer damaged |  | - Confirm you are trained and signed off against this SWI, signed on to JSEA-PRO-MET-201-004 at a communication session, site inducted with current cyanide awareness and gas detection competency where the area requires it, and authorised by the Shift Supervisor. - Confirm a second person is in the area. Cyanide-area sampling is not done alone. - Take 5 minutes before starting (Take 5). Prepare the sampler, carbon strainer and filter paper and check they are in good condition. - Confirm the plant is in steady-state operation - normal operation for a minimum of 15 to 20 minutes before sampling. A sample taken during a swing is not representative. - Notify the control room or DCS operator that the sampling round is starting, and which points it covers. - Confirm the personal gas monitor is on, in calibration date and reading clean, and that fixed detection in the area is healthy. - Confirm the safety shower and eyewash nearest the point are within reach and proven flowing. - Label every container BEFORE sampling - point, date, time and shift. - Rinse the container, cutter or sieve with process water and then with the stream being sampled. Discard the rinse. Equipment Required : - Sampling bucket 5 to 8 L - pH meter - DO meter - Marcy scale - Free cyanide titration kit - standardised AgNO3 - Rhodanine indicator - Burette, pipette, conical flask, stand and wash bottle - Filter press and filter paper - Bottles 250 mL, five sets labelled by tank - Spatula - Distilled water - Carbon strainer, drying oven, tray and digital balance - Personal protective equipment as listed in PPE Requirements |  |  |  |  |  |  |  |  |  |  |  |  |
| - Sample Collection - HCN released at the open sample point - Fall from the tank top or launder platform - Confined space atmosphere at the launder opening - Samples swapped between tanks |  | - Work the train in order from CIL-1 through to CIL-5 so the profile reads in sequence. - At each launder, flush the point and discard the first flow, then collect the sample. - Keep the five samples strictly separate and labelled by tank - a swapped pair inverts the profile and sends the metallurgist the wrong way. CAUTION: Sample from outside the opening only. Do not enter a tank top hatch or launder opening. Carry sample containers in a bag or hoist them, not in the hand on a ladder. |  |  |  |  |  |  |  |  |  |  |  |  |
| - pH and Dissolved Oxygen - Dissolved oxygen reading changed by air exposure - Meter out of calibration - wrong pH |  | - Determine pH and DO on each tank sample as soon as it is taken. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Filtration and Cyanide - Cyanide volatilising from the filtrate - Splash of cyanide-bearing filtrate - Profile sheet incomplete |  | - Filter each sample and titrate free cyanide. - Bottle the laboratory splits for WAD cyanide and assay. - Record all five results on one profile sheet so the trend down the train is visible on the page. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Carbon in the Launder - Sampling, Drying and Weighing - Sampler striking the head - Pinch point at the sampling point cover - Splash of slurry while pouring - Carbon contaminated in the oven |  | - Take the sampler and strainer. Open the sampling point cover carefully. - Put the sampler into the slurry and fill it. Pour the slurry onto the strainer. Do this 5 times. - Close the sampling point cover tightly. - Wash the slurry off the carbon on the strainer with tap water. - Put the washed carbon on filter paper, roll the paper and put it on a tray. Put the tray in the oven on the lowest shelf to avoid contamination. - Take the carbon out of the oven, weigh it on the digital balance and record the weight. - Enter the weight and the carbon concentration on the form G:\Processing\5. Metallurgy\Metallurgy Lab\04 Metlab Data\Carbon\Carbon From Launder. CAUTION: Hard hat on and the position of the sampler watched at all times. Open and close the cover slowly with hands clear of the pinch point. Pour slurry slowly. All HCN controls and PPE for this SWI apply. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Completion and Clean Up - Sample valve left weeping - Samples not handed over with a signed record - Control room not told the round is complete |  | - Close the sample valve or stop the sample pump and confirm the point is not weeping. - Wash down the point and the surrounding walkway. Leave nothing on the grating. - Record sampling time, operator name and the operating conditions read at the time - flow, density and any instrument value available. - Deliver the labelled samples to the laboratory or to the composite station as the schedule requires, and sign the handover. - Notify the control room that the round is complete. - Record the result, the method used and the instrument or balance identity on the field or bench data sheet - date, time, shift, operator and every value determined. Make the register or logbook entry and any handover signature. - Record any step not completed, with the reason. - Report to the Shift Supervisor, and record, any control in Part 2 found not in place. |  |  |  |  |  |  |  |  |  |  |  |  |

| Referenced Documents |  |
| --- | --- |
| Document ID | Document Title |
| JSEA-PRO-MET-201-004 | JSEA — CIL Sampling Round - 032-TK-002 to 032-TK-006 Launders |
| HM-PRC-V01-PRO005 | Carbon in Leach (CIL) Circuit |
| Mt. Morgan site procedure 6 | CIL-1 & 5 Tank Sampling [LIVE] |
| 4034-PR-PRO-002 | Sampling Protocol, CIL table |
| DOC-3-MET-PRS-WIN-00058-IE v1.0 | Martabe WI Sampling Carbon Launder CIL (issued 25/12/2024) - source of Step 5 |
| DOC-2-MET-PRM-SOP-00037-IE | Martabe SOP referenced by the Martabe WI - not received |
| DOC-2-MET-MEL-SOP-00087-IE | Martabe SOP referenced by the Martabe WI - not received |

In the event of an emergency

1. HCN Gas Alarm If the personal monitor or fixed detection alarms (5 ppm alarm, 10 ppm high-high), stop work, leave the area upwind and call CH19 “EMERGENCY”. Do not re-enter until the Shift Supervisor clears the area. 2. Contact with Moving Equipment If a person is caught by or struck by an agitator, pump or screen, hit the nearest emergency stop, call CH19 “EMERGENCY” and call 000. Do not approach the equipment until it is isolated and locked out. 3. Suspected Cyanide Exposure If a person becomes dizzy, short of breath or collapses, call CH19 “EMERGENCY”, call 000, and bring the cyanide antidote kit and a trained first aider. Do not give mouth-to-mouth resuscitation. 4. Skin or Eye Contact with Slurry or Filtrate Flush with copious water at the safety shower or eyewash for a minimum of 15 minutes. Remove contaminated clothing while flushing. Report to the first aider. 5. Person Down at a Tank Top or Launder Opening Do not enter the opening. Call CH19 “EMERGENCY” and call 000. Rescue is by the trained rescue team under the site confined space procedure only. 6. Fall from Height Do not move the injured person unless they are in danger. Call CH19 “EMERGENCY”, call 000, and notify the Shift Supervisor.