Safe Work Instruction

Caustic Soda Strength Titration

This document is the property of Heritage Minerals with all rights are reserved in respect of it. This document cannot be reproduced without the written consent of Heritage Minerals

# Purpose and Scope

This Safe Work Instruction is required so that the mechanical and operational integrity of the system is kept intact. It assists operators and mechanical technicians alike to identify possible problems before they happen.  It will help ensure that when personnel are performing this task it is performed in a safe and secure manner, to ensure the operational integrity of the machine and applies when this task is performed either by itself or in conjunction with other tasks in the area.

A JSA or Take 5 must be completed and discussed with the Supervisor if any variation to this SWI is necessary.

# RESPONSIBILITIES AND AUTHORITY

| What | Who | When |
| --- | --- | --- |
| Conform to procedure: Implement Changes: Change procedure: Review: Authorisation: | All personnel Anyone Training Facilitator Working party of content experts Manager | At all times As required As required On change & yearly On change |

# PPE REQUIREMENTS

- Safety helmet and safety glasses

- Chemical splash goggles and face shield

- High-visibility long-sleeved shirt and long trousers

- Safety boots - nitrile PVC boots

- Chemical resistant suit, acid-resistant and nitrile rubber gloves, and apron

- Personal HCN gas monitor, calibrated and in test date

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

In the event of an emergency

Call on CH19 “EMERGENCY, EMERGENCY, EMERGENCY” on the project to make everyone aware, call 000 for emergency services, police, fire or ambulance, you will need to stay calm and state when requested by a responder:

• Your Name. • Your Location. • Type of Emergency; and • Assistance Required.

| Part 1: Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: |  |  | SWI-PRO-MET-205-002 - Caustic Soda Strength Titration |  |  |  |  |  |  |  |  |  |  |  |
| Issued Date: |  |  | 11/08/2026 |  |  |  |  | Next Review Date: |  |  |  | 11/08/2027 |  |  |
| Document Owner: |  |  | Processing |  |  |  |  | JSEA ID: |  |  |  | JSEA-PRO-MET-205-002 |  |  |
| Part 2: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of work: |  |  |  |  |  | Determine sodium hydroxide strength by acid titration. Caustic strength sets the elution eluant and the pH control across the plant. Standardised HCl is the main titrant. Standardised sulphuric acid, as used at Martabe for ReCYN caustic, is the alternative. Area: 205 Met Lab - Chemical and Leach Testwork. Points and equipment covered: Met lab bench - physical testwork bench and instrument Frequency: Every made-up batch and once per shift on standing solution. Draft for review: Step 4 follows the Martabe work instructions ReCYN Plant Caustic Strength Measurement (DOC-3-MET-PMC-WIN-00088-IE) and Sodium Hydroxide Titration (DOC-3-MET-PMC-WIN-00102-IE), v1.0, 25/12/2024, as an alternative titration with standardised sulphuric acid. HCl titration (Step 3) stays the main method. JSEA-PRO-MET-205-002 must be checked before this instruction is approved. Open items: (1) the Martabe instruction for preparing the sulphuric acid titrant has not been received - set the make-up and standardisation of the titrant before Step 4 is used. |  |  |  |  |  |  |  |  |
| Potential Hazards: |  |  |  |  |  | Contact with sodium hydroxide solution - burn to skin or eyes — Full chemical PPE including face shield and chemical-resistant gloves. Sample drawn slowly from the designated valve. Safety shower and eyewash within reach and proven flowing. Contact with sulphuric or hydrochloric acid - burn to skin or eyes — Full acid PPE including face shield, acid-resistant gloves, apron and boots. Sample drawn only from the designated sample valve. Add acid to water, never the reverse. Safety shower and eyewash within reach and proven flowing. Spill kit at the point. Cuts from broken glassware, and burns from hot glassware — Glassware inspected for chips and cracks before use and discarded if damaged. Hot glassware handled with tongs or heat-resistant gloves and set down on a heat mat. Broken glass to the sharps bin, never to the general waste or by hand. Inhalation of acid fume or reagent vapour at the bench — Work carried out inside the fume cupboard with the sash at the marked working height and the airflow confirmed before starting. Scrubber running and its dosing confirmed. Perchloric work only in the cupboard rated for it, with the washdown routine after every use. No work at an open bench. Splash of cyanide-bearing slurry or solution to skin or eyes — Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Cross-contamination from a container or cutter not cleaned between points — Bucket, container, cutter and sieve rinsed with process water and then with the stream being sampled, and the rinse discarded, before every sample. Dedicated containers for high-grade streams. Labels checked against the schedule before handover. Non-representative sample leading to a wrong process decision — Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. The full step-by-step hazard analysis, with the risk ranking, is in JSEA-PRO-MET-205-002 Section 4. |  |  |  |  |  |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT (PPE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part 3: Safe Work Instruction Steps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Job Number (Hazards/Cautions) |  | Application Method |  |  |  |  |  |  | Visual Reference |  |  |  |  |  |
| - Pre-start Check - Fume cupboard or scrubber not running - Safety shower or eyewash not proven flowing - Instrument out of calibration |  | - Confirm you are trained and signed off against this SWI, signed on to JSEA-PRO-MET-205-002 at a communication session, site inducted with current cyanide awareness and gas detection competency where the area requires it, and authorised by the Shift Supervisor. - Confirm the fume cupboard airflow, the scrubber and its dosing are running before any acid or cyanide work at the bench. - Confirm the safety shower and eyewash are within reach and proven flowing. - Confirm the instrument or balance is within its calibration or verification interval. - Confirm the reagents to be used are in date and correctly labelled. - Inspect all glassware for chips and cracks and discard anything damaged. Equipment Required : - Burette and stand - Pipette and 10 mL syringe - Conical flasks, 100 mL and 500 mL beakers - Standardised HCl - Standardised sulphuric acid (alternative titrant) - Phenolphthalein indicator - Distilled water - Closed 100 to 120 mL sample bottle - Fume cupboard - Personal protective equipment as listed in PPE Requirements |  |  |  |  |  |  |  |  |  |  |  |  |
| - Sampling and Dilution - Concentrated caustic sampled - Caustic splash - Cracked or leaking glassware |  | - Take the sample from the designated diluted-solution valve, never from the concentrated tank. ReCYN caustic is sampled as SWI-PRO-MET-201-020, in a closed 100 to 120 mL bottle. - Check the equipment - no cracked or leaking bottle, beaker or syringe. Confirm the titrant and phenolphthalein are in date. - Dilute the aliquot where the method requires it, adding caustic to water. CAUTION: Add caustic to water, never the reverse. |  |  |  |  |  |  |  |  |  |  |  |  |
| - HCl Titration - Acid splash - End point overshot |  | - Add phenolphthalein indicator - the solution turns pink. - Titrate with standardised HCl until the pink just disappears and stays gone for 30 seconds. - Record the titre and calculate strength. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Sulphuric Acid Titration - Alternative - Sulphuric acid splash - Unstandardised titrant - Wrong reaction ratio |  | - Take 5 mL of the NaOH sample with a syringe and put it in the beaker. Add 3 to 5 drops of phenolphthalein. - Titrate with standardised sulphuric acid from the burette, swirling gently, until the pink just disappears and stays gone for 30 seconds. The Martabe "soft pink" end point is not used. - Use the standardised molarity of the sulphuric acid. A "5 per cent" acid is not a molarity and is standardised before use. - Sulphuric acid reacts 2:1 - H2SO4 + 2 NaOH gives Na2SO4 + 2 H2O. Calculate [NaOH] = 2 x [H2SO4] x V(H2SO4) / V(sample NaOH). Multiply [NaOH] in mol/L by 40.0 for g/L. CAUTION: Sulphuric acid reacts 2:1 with NaOH. Use the factor 2, not the HCl calculation. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Reporting and Waste - Off-strength batch used - Waste to the wrong stream |  | - Compare against the target make-up strength and report any batch outside range before it is used. - Pour the remaining solution to the site waste stream for its classification. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Completion and Clean Up - Cyanide-bearing residue to the general drain - Glassware left dirty - Result not traceable to the instrument |  | - Wash down the bench and the fume cupboard working surface. - Clean glassware and return it to the drying rack. - Dispose of residues to the correct waste stream - cyanide-bearing effluent does NOT go to the general drain. - Record the result, the method used and the instrument or balance identity. - Record every value determined on the field or bench data sheet - date, time, shift and operator. Make the register or logbook entry and any handover signature. - Record any step not completed, with the reason. - Report to the Shift Supervisor, and record, any control in Part 2 found not in place. |  |  |  |  |  |  |  |  |  |  |  |  |

| Referenced Documents |  |
| --- | --- |
| Document ID | Document Title |
| JSEA-PRO-MET-205-002 | JSEA — Caustic Soda Strength Titration |
| KBK-MIR-MP-PRO-OPE-SOP-0009 | Titrasi Kaustik Soda |
| KBK-MIR-MP-PRO-MET-SOP-0025 and SOP-0039/0040 | Hydrochloric acid titration (received 11 Aug 2026) |
| DOC-3-MET-PMC-WIN-00088-IE v1.0 | Martabe WI ReCYN Plant Caustic Strength Measurement (25/12/2024) - source of the Step 4 calculation |
| DOC-3-MET-PMC-WIN-00102-IE v1.0 | Martabe WI Sodium Hydroxide Titration (25/12/2024) - source of the Step 4 titration |
| SWI-PRO-MET-201-020 | Reagent Solution Strength Sampling - ReCYN caustic sample |

In the event of an emergency

1. Skin or Eye Contact with Reagent Flush with copious water at the safety shower or eyewash for a minimum of 15 minutes. Remove contaminated clothing while flushing. Report to the first aider. 2. HCl Gas or Acid Contact Leave the area upwind. Flush skin or eyes with copious water at the safety shower or eyewash for a minimum of 15 minutes. If breathing is affected call CH19 “EMERGENCY” and call 000. 3. Cuts from Broken Glassware Apply pressure to stop the bleeding, get a trained first aider, and report to the Shift Supervisor. Call CH19 “EMERGENCY” and 000 for a serious cut.