Safe Work Procedure

Metallurgical Accounting and Reconciliation

SWP-206

| DOCUMENT HISTORY AND STATUS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Document: | Metallurgical Accounting and Reconciliation Safe Work Procedure |  |  | Version No.: | A |  |  |
| Document No: | SWP-206 |  |  | Owner: | Process Operations Superintendent |  |  |
| Issue Date: | 11/08/2026 |  |  | Date Reviewed: | - |  |  |
| History: | Process owner: Heritage Mineral Operations, Business unit Mt. Morgan. Document approver: Process Manager. |  |  |  |  |  |  |
| Date Reviewed: | Version No.: | Written by: |  | Reviewed by: |  | Authorised by: |  |
| 11/08/2026 | A | GGT (cost code 4034-033) |  |  |  |  |  |
| Revision Description: |  |  |  |  |  |  |  |
| Changes: |  |  |  |  |  |  |  |
| Rev A - first issue. Converted from the GGT Standard Work Procedure layout to the Heritage Safe Work Procedure format; content unchanged. |  |  |  |  |  |  |  |
| Action: | Name: |  | Position: |  |  |  | Date: |
| Prepared by | GGT (cost code 4034-033) |  | Document preparer |  |  |  | 11/08/2026 |
| Reviewed by |  |  | Process Operations Superintendent |  |  |  |  |
| Reviewed by |  |  |  |  |  |  |  |
| Authorised by |  |  | Process Manager |  |  |  |  |

This document is controlled electronically as part of Heritage Minerals Pty Limited’s Health & Safety Management System.  Please refer to Heritage Minerals Controlled Document Register for the current version.

Table of Contents

1.	Purpose and Scope	3

2.	Responsibilities	3

3.	Abbreviations and Definitions	3

4.	Potential Hazards	4

5.	Personal Protective Equipment	4

6.	Procedure	5

6.1	Introduction	5

6.2	Procedure and Implementation Steps	5

6.3	Schedule	5

6.4	Flowchart	6

7.	Training	6

8.	Document Traceability	6

9.	Review Criteria	7

10.	Legislation & References	7

10.1	Legislation	7

10.2	Source Documents	7

# Purpose and Scope

This procedure covers metallurgical accounting and reconciliation at the Mt. Morgan processing plant.

Metallurgical accounting and reconciliation - daily and monthly mass balance, circuit inventory and lock-up, carbon and resin movement accounting, reagent consumption, bullion and concentrate reconciliation, and the investigation of a balance that does not close.

Document details:

Hazard analyses beneath: 12 (JSEA-PRO-MET-202-001 to -012)

Work instructions beneath: 0

Process owner: Heritage Mineral Operations, Business unit Mt. Morgan

Document owner: Process Operations Superintendent

Document approver: Process Manager

# Responsibilities

| Staff | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician / Met Lab Technician | - Carry out the task to the schedule and to the work instruction. - Only carry out work where competent and authorised to do so. - Record what was actually done. Work not done is recorded as not done - never invented and never back-filled. - Stop and report if any control in the JSEA is not in place. |
| Shift Supervisor / Coordinator | - Authorise the task and confirm the area is fit before it starts. - Know that the task is in progress and where the person is. - Understand that much of the plant is automated and equipment may start or stop without warning. - Act on any off-specification result reported from the task. |
| Metallurgist | - Set and maintain the schedule - what is done, how often, and what is reported. - Review the results and act on the trends. - Investigate any variance outside the agreed tolerance before a period is closed. |
| Process Manager / Superintendent | - Ensure all personnel are trained and competent. - Ensure this procedure and the documents beneath it are reviewed at the stated interval. - Approve the risk ranking in the JSEAs. |

# Abbreviations and Definitions

| Term | Meaning |
| --- | --- |
| JSEA | Job safety and environmental analysis - one per task, giving the step-by-step hazards, controls and risk ranking. |
| SWI | Safe work instruction - one per task, giving the step-by-step method. |
| SWP | Safe work procedure - this document. |
| Steady state | Normal operation held for at least 15 to 20 minutes. |
| ReCYN | The resin-in-pulp cyanide and metal recovery process in Areas 041, 051, 071 and 081. |
| CN Free / WAD CN | Free cyanide by silver nitrate titration; weak acid dissociable cyanide by distillation. |

# Potential Hazards

The step-by-step hazard analysis for each task, with the risk ranking, is in the JSEA for that task. The hazards common across this area are:

An accounting or reconciliation error that hides a real metal loss

Non-representative sample leading to a wrong process decision

Exposure to a gold security incident, and to suspicion in one

Mercury vapour in the gold room, elution and retort areas

Hydrogen cyanide gas released at the open sample point

Fall from a tank top, launder platform, column platform or access ladder

Manual handling injury from carrying sample buckets, containers and composite drums

Cross-contamination from a container or cutter not cleaned between points

# Personal Protective Equipment

Standard PPE is worn for all normal operations: hard hat, safety glasses, high-visibility long-sleeved shirt and long trousers, safety boots, and gloves appropriate to the task.

The following is added where the task requires it:

Personal HCN gas monitor, calibrated and in test date

Chemical splash goggles and face shield

Nitrile rubber gloves

Nitrile PVC boots

Chemical resistant suit

Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

Harness and fall arrest where no fixed platform exists

# Procedure

## Introduction

| Warning: | many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times. |
| --- | --- |

## Procedure and Implementation Steps

The documents below are the controlled method for each task. This procedure does not repeat them.

| JSEA | Work instruction | Task | Class |
| --- | --- | --- | --- |
| JSEA-PRO-MET-202-001 | - | Daily Metallurgical Accounting and Plant Mass Balance | Operating |
| JSEA-PRO-MET-202-002 | - | Shift and Daily Production Data Entry and Validation | Operating |
| JSEA-PRO-MET-202-003 | - | Monthly Metal Accounting and Reconciliation | Operating |
| JSEA-PRO-MET-202-004 | - | Gold and Silver Circuit Inventory and Lock-Up Reconciliation | Safety-critical |
| JSEA-PRO-MET-202-005 | - | Carbon Inventory and Carbon Movement Accounting | Operating |
| JSEA-PRO-MET-202-006 | - | Resin Inventory and Resin Movement Accounting | Operating |
| JSEA-PRO-MET-202-007 | - | Reagent Consumption Tracking and Reconciliation | Operating |
| JSEA-PRO-MET-202-008 | - | Bullion Reconciliation Against Refinery Outturn | Safety-critical |
| JSEA-PRO-MET-202-009 | - | Copper Concentrate Lot Accounting and Dispatch Reconciliation | Operating |
| JSEA-PRO-MET-202-010 | - | Sample Dispatch, Umpire Assay and External Laboratory Reconciliation | Operating |
| JSEA-PRO-MET-202-011 | - | Metallurgical Report Preparation and Distribution | Operating |
| JSEA-PRO-MET-202-012 | - | Investigation and Correction of Mass Balance Discrepancy | Operating |

## Schedule

| Task | Frequency |
| --- | --- |
| 202-001 Daily Metallurgical Accounting and Plant Mass Balance | Daily, closed before the morning production meeting. |
| 202-002 Shift and Daily Production Data Entry and Validation | Every shift, at handover. |
| 202-003 Monthly Metal Accounting and Reconciliation | Monthly, within the agreed reporting deadline. |
| 202-004 Gold and Silver Circuit Inventory and Lock-Up Reconciliation | Monthly at the close, and after any circuit shutdown or major upset. |
| 202-005 Carbon Inventory and Carbon Movement Accounting | Every carbon movement, reconciled monthly. |
| 202-006 Resin Inventory and Resin Movement Accounting | Every resin transfer, reconciled monthly. |
| 202-007 Reagent Consumption Tracking and Reconciliation | Daily entry, reconciled monthly against deliveries and stock. |
| 202-008 Bullion Reconciliation Against Refinery Outturn | Every dispatch, reconciled at the monthly close. |
| 202-009 Copper Concentrate Lot Accounting and Dispatch Reconciliation | Every lot. |
| 202-010 Sample Dispatch, Umpire Assay and External Laboratory Reconciliation | Every dispatch; reconciliation monthly. |
| 202-011 Metallurgical Report Preparation and Distribution | Daily, weekly and monthly per the reporting calendar. |
| 202-012 Investigation and Correction of Mass Balance Discrepancy | Whenever a balance falls outside the agreed tolerance. |

## Flowchart

Not reproduced here. Each work instruction carries its own step sequence.

# Training

Every person carrying out a task in this area is trained and signed off against the work instruction for that task, and has signed on to the matching JSEA at a communication session.

Site induction, current.

Cyanide awareness and gas detection competency where the area requires it.

Working at height competency where the task requires it.

Competency in the determinations, instruments and calculations used in the task.

Refresher at the interval set by the site training matrix, and after any change to the instruction.

# Document Traceability

The three-digit suffix is shared between the hazard analysis and the work instruction, so SWI-PRO-MET-202-011 is the work instruction for JSEA-PRO-MET-202-011.

| Item | Count |
| --- | --- |
| Tasks in area 202 | 12 |
| Hazard analyses issued | 12 |
| Work instructions issued | 0 |
| Desk-based tasks with a JSEA but no work instruction | 12 |
| Safety-critical | 2 |
| Issued but NOT FOR USE | 1 |

# Review Criteria

This Procedure and the documents beneath it shall be reviewed as follows:

Every 12 months; or

Immediately after any incident, plant change, equipment change or change to the schedule.

# Legislation & References

## Legislation

Queensland Mining and Quarrying Safety and Health Act 1999

Queensland Mining and Quarrying Safety and Health Regulation 2017

## Source Documents

The source documents and notes held for the tasks in this area.

No direct source held. Written from first principles against the plant configuration.

MM-4034-033-P-09 Tag Register (942 tags) and the mechanical equipment list

4034-PR-PRO-002 Sampling Protocol - the sample points the balance is built on

[LIVE] SOP-Mt.Morgan-Data Entry-xxx.docx and its change form template

Mt. Morgan Changeform Template

No direct source held. Written from first principles.

RA-01 Resin Loss Register and RA-02 Carbon Contamination Register, both Rev A

Mechanical equipment list - vessel volumes

MM-4034-033-RES Resin Line Schedule (268 lines)

KBK-MIR-MP-PRO-MET-SOP-0044 Pengiriman Bullion, received 11 Aug 2026

RA-02 Carbon Contamination Register Rev A and its study report

Mechanical equipment list

KBK-MIR-MP-PRO-OPE-SOP-0013 Particle Size Distribution of Activated Carbon

RA-01 Resin Loss Register Rev A and its study report

KBK-MIR-MP-PRO-MET-SOP-0058 Distribusi Ukuran Partikel Resin and KBK SOP-PROC-CNREC-12 Fresh resin conditioning, both received 11 Aug 2026

HM-PP-006A Reagent First Fill and HM-PP-006B Reagents Consumption 2026

Mt. Morgan Reagent Consumption and Pricing folder, including the pricing reference set

HM-PRC-V01-PRO021 to PRO027 reagent procedures

KBK-MIR-MP-PRO-MET-SOP-0044 Pengiriman Bullion, received 11 Aug 2026 - covers dispatch, weighing, packing, documentation and handover

No source held for the outturn reconciliation half of this task

No source held. HM_PP_009 Product Quality Management System folder arrived empty.

KBK-MIR-LAB-PRE-SOP-009 Pengiriman Sampel ke laboratorium eksternal

4034-PR-PRO-002 Sampling Protocol

RA-01 Resin Loss Register and RA-02 Carbon Contamination Register