Safe Work Procedure

Plant Surveys and Circuit Performance Assessment

SWP-207

| DOCUMENT HISTORY AND STATUS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Document: | Plant Surveys and Circuit Performance Assessment Safe Work Procedure |  |  | Version No.: | A |  |  |
| Document No: | SWP-207 |  |  | Owner: | Process Operations Superintendent |  |  |
| Issue Date: | 11/08/2026 |  |  | Date Reviewed: | - |  |  |
| History: | Process owner: Heritage Mineral Operations, Business unit Mt. Morgan. Document approver: Process Manager. |  |  |  |  |  |  |
| Date Reviewed: | Version No.: | Written by: |  | Reviewed by: |  | Authorised by: |  |
| 11/08/2026 | A | GGT (cost code 4034-033) |  |  |  |  |  |
| Revision Description: |  |  |  |  |  |  |  |
| Changes: |  |  |  |  |  |  |  |
| Rev A - first issue. Converted from the GGT Standard Work Procedure layout to the Heritage Safe Work Procedure format; content unchanged. |  |  |  |  |  |  |  |
| Action: | Name: |  | Position: |  |  |  | Date: |
| Prepared by | GGT (cost code 4034-033) |  | Document preparer |  |  |  | 11/08/2026 |
| Reviewed by |  |  | Process Operations Superintendent |  |  |  |  |
| Reviewed by |  |  |  |  |  |  |  |
| Authorised by |  |  | Process Manager |  |  |  |  |

This document is controlled electronically as part of Heritage Minerals Pty Limited’s Health & Safety Management System.  Please refer to Heritage Minerals Controlled Document Register for the current version.

Table of Contents

1.	Purpose and Scope	3

2.	Responsibilities	3

3.	Abbreviations and Definitions	3

4.	Potential Hazards	4

5.	Personal Protective Equipment	4

6.	Procedure	5

6.1	Introduction	5

6.2	Procedure and Implementation Steps	5

6.3	Schedule	6

6.4	Flowchart	6

7.	Training	6

8.	Document Traceability	7

9.	Review Criteria	7

10.	Legislation & References	7

10.1	Legislation	7

10.2	Source Documents	7

# Purpose and Scope

This procedure covers plant surveys and circuit performance assessment at the Mt. Morgan processing plant.

Plant surveys and circuit performance assessment - survey planning and execution, flow and density verification, screen and cyclone performance, residence time, leach, CIL and adsorption profiles, thickener performance, elution assessment, dosing verification, screen efficiency, carbon and resin loss, and oxygen utilisation.

Document details:

Hazard analyses beneath: 12 (JSEA-PRO-MET-203-001 to -012)

Work instructions beneath: 12

Process owner: Heritage Mineral Operations, Business unit Mt. Morgan

Document owner: Process Operations Superintendent

Document approver: Process Manager

# Responsibilities

| Staff | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician / Met Lab Technician | - Carry out the task to the schedule and to the work instruction. - Only carry out work where competent and authorised to do so. - Record what was actually done. Work not done is recorded as not done - never invented and never back-filled. - Stop and report if any control in the JSEA is not in place. |
| Shift Supervisor / Coordinator | - Authorise the task and confirm the area is fit before it starts. - Know that the task is in progress and where the person is. - Understand that much of the plant is automated and equipment may start or stop without warning. - Act on any off-specification result reported from the task. |
| Metallurgist | - Set and maintain the schedule - what is done, how often, and what is reported. - Review the results and act on the trends. - Investigate any variance outside the agreed tolerance before a period is closed. |
| Process Manager / Superintendent | - Ensure all personnel are trained and competent. - Ensure this procedure and the documents beneath it are reviewed at the stated interval. - Approve the risk ranking in the JSEAs. |

# Abbreviations and Definitions

| Term | Meaning |
| --- | --- |
| JSEA | Job safety and environmental analysis - one per task, giving the step-by-step hazards, controls and risk ranking. |
| SWI | Safe work instruction - one per task, giving the step-by-step method. |
| SWP | Safe work procedure - this document. |
| Steady state | Normal operation held for at least 15 to 20 minutes. |
| ReCYN | The resin-in-pulp cyanide and metal recovery process in Areas 041, 051, 071 and 081. |
| CN Free / WAD CN | Free cyanide by silver nitrate titration; weak acid dissociable cyanide by distillation. |

# Potential Hazards

The step-by-step hazard analysis for each task, with the risk ranking, is in the JSEA for that task. The hazards common across this area are:

Hydrogen cyanide gas released at the open sample point

Splash of cyanide-bearing slurry or solution to skin or eyes

Fall from a tank top, launder platform, column platform or access ladder

Working alone in a cyanide area where a collapse would not be seen

Contact with an agitator, sample cutter, pump or screen that starts without warning

Slip, trip or fall on wet or slurry-covered walkways around sample points

Non-representative sample leading to a wrong process decision

Cross-contamination from a container or cutter not cleaned between points

Manual handling injury from carrying sample buckets, containers and composite drums

Hearing damage in the mill, screen, compressor and blower areas

Inhalation of or contact with solid reagent - cyanide briquettes, lime or salt

Confined space atmosphere at a tank top hatch, launder or hopper opening

Contact with hot solution or hot surface at the elution, stripping or regeneration circuits

Mercury vapour in the gold room, elution and retort areas

# Personal Protective Equipment

Standard PPE is worn for all normal operations: hard hat, safety glasses, high-visibility long-sleeved shirt and long trousers, safety boots, and gloves appropriate to the task.

The following is added where the task requires it:

Personal HCN gas monitor, calibrated and in test date

Chemical splash goggles and face shield

Nitrile rubber gloves

Nitrile PVC boots

Chemical resistant suit

Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

Harness and fall arrest where no fixed platform exists

Hearing protection

Heat-resistant gloves

Face shield

Acid-resistant gloves, apron and face shield

Chemical splash goggles

Personal multi-gas monitor including H2S, calibrated and in test date

# Procedure

## Introduction

| Warning: | many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times. |
| --- | --- |

## Procedure and Implementation Steps

The documents below are the controlled method for each task. This procedure does not repeat them.

| JSEA | Work instruction | Task | Class |
| --- | --- | --- | --- |
| JSEA-PRO-MET-203-001 | SWI-PRO-MET-203-001 | Plant Survey Planning, Permitting and Execution | Safety-critical |
| JSEA-PRO-MET-203-002 | SWI-PRO-MET-203-002 | Slurry Flow, Density and Tonnage Verification Survey | Safety-critical |
| JSEA-PRO-MET-203-003 | SWI-PRO-MET-203-003 | Screen and Cyclone Performance Survey - Size Split Determination | Safety-critical |
| JSEA-PRO-MET-203-004 | SWI-PRO-MET-203-004 | Residence Time and Tracer Test on Leach and Adsorption Trains | Safety-critical |
| JSEA-PRO-MET-203-005 | SWI-PRO-MET-203-005 | Leach and CIL Profile Survey - Tank-by-Tank Gold and Cyanide Profile | Safety-critical |
| JSEA-PRO-MET-203-006 | SWI-PRO-MET-203-006 | Adsorption Profile Survey - Metal and Cyanide Resin Circuits | Safety-critical |
| JSEA-PRO-MET-203-007 | SWI-PRO-MET-203-007 | Thickener Performance Survey - Flux, Bed Level and Flocculant Response | Safety-critical |
| JSEA-PRO-MET-203-008 | SWI-PRO-MET-203-008 | Elution and Electrowinning Batch Performance Assessment | Safety-critical |
| JSEA-PRO-MET-203-009 | SWI-PRO-MET-203-009 | Reagent Dosing Verification and Dosing Pump Calibration Check | Safety-critical |
| JSEA-PRO-MET-203-010 | SWI-PRO-MET-203-010 | Interstage and Safety Screen Efficiency Check and Loss Quantification | Safety-critical |
| JSEA-PRO-MET-203-011 | SWI-PRO-MET-203-011 | Carbon and Resin Loss Quantification Survey | Safety-critical |
| JSEA-PRO-MET-203-012 | SWI-PRO-MET-203-012 | Oxygen Utilisation and Sparger Performance Survey | Safety-critical |

## Schedule

| Task | Frequency |
| --- | --- |
| 203-001 Plant Survey Planning, Permitting and Execution | As required for a performance survey; typically quarterly and after any significant plant change. |
| 203-002 Slurry Flow, Density and Tonnage Verification Survey | Quarterly, and after any instrument replacement or recalibration. |
| 203-003 Screen and Cyclone Performance Survey - Size Split Determination | Quarterly, and after any panel change or aperture change. |
| 203-004 Residence Time and Tracer Test on Leach and Adsorption Trains | Once at commissioning, then after any change to feed rate, agitator or tank configuration. |
| 203-005 Leach and CIL Profile Survey - Tank-by-Tank Gold and Cyanide Profile | Quarterly, and whenever recovery moves without an obvious cause. |
| 203-006 Adsorption Profile Survey - Metal and Cyanide Resin Circuits | Quarterly, and after any change to resin inventory or transfer rate. |
| 203-007 Thickener Performance Survey - Flux, Bed Level and Flocculant Response | Quarterly, and whenever overflow clarity or underflow density moves. |
| 203-008 Elution and Electrowinning Batch Performance Assessment | Every batch, assessed and trended monthly. |
| 203-009 Reagent Dosing Verification and Dosing Pump Calibration Check | Quarterly, and after any dosing pump repair or replacement. |
| 203-010 Interstage and Safety Screen Efficiency Check and Loss Quantification | Monthly, and after any panel replacement. |
| 203-011 Carbon and Resin Loss Quantification Survey | Quarterly, and whenever the inventory reconciliation shows an unexplained loss. |
| 203-012 Oxygen Utilisation and Sparger Performance Survey | Quarterly, and after any sparger clean or replacement. |

## Flowchart

Not reproduced here. Each work instruction carries its own step sequence.

# Training

Every person carrying out a task in this area is trained and signed off against the work instruction for that task, and has signed on to the matching JSEA at a communication session.

Site induction, current.

Cyanide awareness and gas detection competency where the area requires it.

Working at height competency where the task requires it.

Competency in the determinations, instruments and calculations used in the task.

Refresher at the interval set by the site training matrix, and after any change to the instruction.

# Document Traceability

The three-digit suffix is shared between the hazard analysis and the work instruction, so SWI-PRO-MET-203-011 is the work instruction for JSEA-PRO-MET-203-011.

| Item | Count |
| --- | --- |
| Tasks in area 203 | 12 |
| Hazard analyses issued | 12 |
| Work instructions issued | 12 |
| Desk-based tasks with a JSEA but no work instruction | 0 |
| Safety-critical | 12 |
| Issued but NOT FOR USE | 0 |

# Review Criteria

This Procedure and the documents beneath it shall be reviewed as follows:

Every 12 months; or

Immediately after any incident, plant change, equipment change or change to the schedule.

# Legislation & References

## Legislation

Queensland Mining and Quarrying Safety and Health Act 1999

Queensland Mining and Quarrying Safety and Health Regulation 2017

## Source Documents

The source documents and notes held for the tasks in this area.

KBK-MIR-MP-PRO-MET-SOP-0027 Grinding Survey, received 11 Aug 2026 - a worked plant survey end to end

KBK-MIR-MP-PRO-MET-SOP-0047 Pengukuran Profil Lumpur Cyclone Over Flow, received 11 Aug 2026

4034-PR-PRO-002 Sampling Protocol

KBK-MIR-MP-PRO-OPE-SOP-0004 SG Determination by Marcy Scale

KBK-MIR-MP-PRO-MET-SOP-0045 Pengukuran Densitas Lumpur, received 11 Aug 2026

4034-002-VE-025 McLanahan rotary scrubber IO&M and 4034-001-VE-038 IsaMill control philosophy, both received 11 Aug 2026

KBK-MIR-MP-PRO-OPE-SOP-0008 Cyclone Efficiency

KBK-MIR-MP-PRO-MET-SOP-0029 Sizing Harian COF and SOP-0020 Distribusi Logam dan Ukuran, both received 11 Aug 2026

MM-4034-033 Screen Aperture Register

4034-004-VE-025 and 4034-011-VE-022 Goldquip screen manuals, received 11 Aug 2026

No direct source held. Written from first principles.

Plant Operating Manual Sections 3.4 and 3.5 - train configuration and design residence time

4034-PR-PRO-002 Sampling Protocol, Leach Area table

KBK-MIR-MP-PRO-MET-SOP-0019 Profil Leaching and SOP-0048 leach tank sampling and profile, both received 11 Aug 2026

HM-PRC-V01-PRO004 Leach Circuit and PRO005 Carbon in Leach

4034-PR-PRO-002 Sampling Protocol, ReCYN Adsorption table

KBK-MIR-MP-PRO-MET-SOP-0049 resin tank sampling and profile and SOP-0058 resin PSD, both received 11 Aug 2026

KBK SOP-PROC-CNREC-01 Resin transfer between adsorption tanks, received 11 Aug 2026

HM-PRC-V01-PRO006 and PRO007

Tail Thickener and Cu Thickener Field Operator Procedures

4034-018A-VE-007 Roytec tails thickener control philosophy and 4034-018A-VE-045 IOM, both received 11 Aug 2026 - rake torque, bed level and underflow density limits

4034-019-VE-006 Model 01 and 4034-019B-VE-007 Model 05 Roytec flocculant control philosophies, received 11 Aug 2026

KBK-MIR-MP-PRO-OPE-SOP-0014 Settling Test

HM-PRC-V01-PRO008 and PRO016

4034-013-VE-028 Applied Heat Thermomat 2300 O&M manual, received 11 Aug 2026 - strip temperatures and circuit design conditions

4034-014-VE-004 and -019 Stantil carbon regeneration kiln control philosophy and IOM, received 11 Aug 2026

HM-PRC-V01-PRO011 Gold Elution and PRO013 Gold Electrowinning

HM-PRC-V01-PRO021 to PRO027 reagent procedures

HM-PP-006B Reagents Consumption 2026 - design dose rates

4034-046-VE-019 Transmin quicklime slaking plant functional description, received 11 Aug 2026

RA-01 Resin Loss and RA-02 Carbon Contamination Registers Rev A

KBK-MIR-MP-PRO-MET-SOP-0028 Sizing Harian CIL Tail, received 11 Aug 2026

SWI-PRO-OPR-032-003, -004 and -005 screen instructions, issued

RA-01 Resin Loss Register and RA-02 Carbon Contamination Register Rev A, and their study reports

MM-4034-033-RES Resin Line Schedule (268 lines)

KBK-MIR-MP-PRO-MET-SOP-0058 Distribusi Ukuran Partikel Resin, received 11 Aug 2026

4034-044-VE-003 Rev B Glencore Hypersparge supersonic gas sparger installation and operating instructions, received 11 Aug 2026

KBK-MIR-MP-PRO-MET-SOP-0046 Pengukuran Oksigen Terlarut, received 11 Aug 2026

HM-PRC-V01-PRO004 Leach Circuit and SWI-PRO-OPR-031-004 Oxygen Sparger Inspection and Cleaning