Safe Work Procedure

Automatic Sampling and Online Analyser Operation

SWP-211

| DOCUMENT HISTORY AND STATUS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Document: | Automatic Sampling and Online Analyser Operation Safe Work Procedure |  |  | Version No.: | A |  |  |
| Document No: | SWP-211 |  |  | Owner: | Process Operations Superintendent |  |  |
| Issue Date: | 11/08/2026 |  |  | Date Reviewed: | - |  |  |
| History: | Process owner: Heritage Mineral Operations, Business unit Mt. Morgan. Document approver: Process Manager. |  |  |  |  |  |  |
| Date Reviewed: | Version No.: | Written by: |  | Reviewed by: |  | Authorised by: |  |
| 11/08/2026 | A | GGT (cost code 4034-033) |  |  |  |  |  |
| Revision Description: |  |  |  |  |  |  |  |
| Changes: |  |  |  |  |  |  |  |
| Rev A - first issue. Converted from the GGT Standard Work Procedure layout to the Heritage Safe Work Procedure format; content unchanged. |  |  |  |  |  |  |  |
| Action: | Name: |  | Position: |  |  |  | Date: |
| Prepared by | GGT (cost code 4034-033) |  | Document preparer |  |  |  | 11/08/2026 |
| Reviewed by |  |  | Process Operations Superintendent |  |  |  |  |
| Reviewed by |  |  |  |  |  |  |  |
| Authorised by |  |  | Process Manager |  |  |  |  |

This document is controlled electronically as part of Heritage Minerals Pty Limited’s Health & Safety Management System.  Please refer to Heritage Minerals Controlled Document Register for the current version.

Table of Contents

1.	Purpose and Scope	3

2.	Responsibilities	3

3.	Abbreviations and Definitions	3

4.	Potential Hazards	4

5.	Personal Protective Equipment	4

6.	Procedure	5

6.1	Introduction	5

6.2	Procedure and Implementation Steps	5

6.3	Schedule	5

6.4	Flowchart	6

7.	Training	6

8.	Document Traceability	6

9.	Review Criteria	7

10.	Legislation & References	7

10.1	Legislation	7

10.2	Source Documents	7

# Purpose and Scope

This procedure covers automatic sampling and online analyser operation at the Mt. Morgan processing plant.

Automatic sampling and online analyser operation - the five automatic samplers, the two online cyanide analysers, their calibration, servicing and alarm response, and the bias and correlation checks that prove they are telling the truth.

Document details:

Hazard analyses beneath: 12 (JSEA-PRO-MET-207-001 to -012)

Work instructions beneath: 11

Process owner: Heritage Mineral Operations, Business unit Mt. Morgan

Document owner: Process Operations Superintendent

Document approver: Process Manager

# Responsibilities

| Staff | Responsibilities |
| --- | --- |
| Operator / Metallurgical Technician / Met Lab Technician | - Carry out the task to the schedule and to the work instruction. - Only carry out work where competent and authorised to do so. - Record what was actually done. Work not done is recorded as not done - never invented and never back-filled. - Stop and report if any control in the JSEA is not in place. |
| Shift Supervisor / Coordinator | - Authorise the task and confirm the area is fit before it starts. - Know that the task is in progress and where the person is. - Understand that much of the plant is automated and equipment may start or stop without warning. - Act on any off-specification result reported from the task. |
| Metallurgist | - Set and maintain the schedule - what is done, how often, and what is reported. - Review the results and act on the trends. - Investigate any variance outside the agreed tolerance before a period is closed. |
| Process Manager / Superintendent | - Ensure all personnel are trained and competent. - Ensure this procedure and the documents beneath it are reviewed at the stated interval. - Approve the risk ranking in the JSEAs. |

# Abbreviations and Definitions

| Term | Meaning |
| --- | --- |
| JSEA | Job safety and environmental analysis - one per task, giving the step-by-step hazards, controls and risk ranking. |
| SWI | Safe work instruction - one per task, giving the step-by-step method. |
| SWP | Safe work procedure - this document. |
| Steady state | Normal operation held for at least 15 to 20 minutes. |
| ReCYN | The resin-in-pulp cyanide and metal recovery process in Areas 041, 051, 071 and 081. |
| CN Free / WAD CN | Free cyanide by silver nitrate titration; weak acid dissociable cyanide by distillation. |

# Potential Hazards

The step-by-step hazard analysis for each task, with the risk ranking, is in the JSEA for that task. The hazards common across this area are:

Hydrogen cyanide gas released at the open sample point

Splash of cyanide-bearing slurry or solution to skin or eyes

Contact with an agitator, sample cutter, pump or screen that starts without warning

Contact with live electrical equipment at a sample pump, cutter or local control station

Non-representative sample leading to a wrong process decision

Release from a pressurised sample line, hose or fitting

Fall from a tank top, launder platform, column platform or access ladder

Cross-contamination from a container or cutter not cleaned between points

Inhalation of acid fume or reagent vapour at the bench

Cuts from broken glassware, and burns from hot glassware

Confined space atmosphere at a tank top hatch, launder or hopper opening

# Personal Protective Equipment

Standard PPE is worn for all normal operations: hard hat, safety glasses, high-visibility long-sleeved shirt and long trousers, safety boots, and gloves appropriate to the task.

The following is added where the task requires it:

Personal HCN gas monitor, calibrated and in test date

Chemical splash goggles and face shield

Nitrile rubber gloves

Nitrile PVC boots

Chemical resistant suit

Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

Harness and fall arrest where no fixed platform exists

# Procedure

## Introduction

| Warning: | many operations on the gold processing plant are automated and equipment may start or stop without warning. It is the operator's responsibility to ensure the safety of all personnel working on the plant at all times. |
| --- | --- |

## Procedure and Implementation Steps

The documents below are the controlled method for each task. This procedure does not repeat them.

| JSEA | Work instruction | Task | Class |
| --- | --- | --- | --- |
| JSEA-PRO-MET-207-001 | SWI-PRO-MET-207-001 | Automatic Sampler Operation and Cut Verification - IsaMill Feed 022-XM-011 and 022-XM-017 | Safety-critical |
| JSEA-PRO-MET-207-002 | SWI-PRO-MET-207-002 | Automatic Sampler Operation and Cut Verification - CIL Tails 032-XM-012 | Safety-critical |
| JSEA-PRO-MET-207-003 | SWI-PRO-MET-207-003 | Automatic Sampler Operation and Cut Verification - Metal Adsorption Tails 041-XM-013 | Safety-critical |
| JSEA-PRO-MET-207-004 | SWI-PRO-MET-207-004 | Automatic Sampler Operation and Cut Verification - Final Tails 051-XM-014 | Safety-critical |
| JSEA-PRO-MET-207-005 | SWI-PRO-MET-207-005 | Automatic Sampler Cleaning, Unblocking and Cutter Wear Check | Safety-critical |
| JSEA-PRO-MET-207-006 | SWI-PRO-MET-207-006 | Automatic versus Manual Sample Bias Check and Reconciliation | Safety-critical |
| JSEA-PRO-MET-207-007 | SWI-PRO-MET-207-007 | Cyanide Analyser Daily Operation and Check - 032-CA-001 and 051-CA-002 | Safety-critical |
| JSEA-PRO-MET-207-008 | SWI-PRO-MET-207-008 | Cyanide Analyser Calibration and Standardisation | Safety-critical |
| JSEA-PRO-MET-207-009 | SWI-PRO-MET-207-009 | Cyanide Analyser Specialised Electrode Replacement and Conditioning | Safety-critical |
| JSEA-PRO-MET-207-010 | SWI-PRO-MET-207-010 | Cyanide Analyser Sample and Booster Pump Service - 032-PP-207/208 and 041-PP-2xx | Safety-critical |
| JSEA-PRO-MET-207-011 | SWI-PRO-MET-207-011 | Cyanide Analyser Leak Detector and High Level Alarm Response | Safety-critical |
| JSEA-PRO-MET-207-012 | - | Analyser to Laboratory Correlation Check and Drift Investigation | Safety-critical |

## Schedule

| Task | Frequency |
| --- | --- |
| 207-001 Automatic Sampler Operation and Cut Verification - IsaMill Feed 022-XM-011 and 022-XM-017 | To be set once the sampler design is known. |
| 207-002 Automatic Sampler Operation and Cut Verification - CIL Tails 032-XM-012 | To be set once the sampler design is known. |
| 207-003 Automatic Sampler Operation and Cut Verification - Metal Adsorption Tails 041-XM-013 | To be set once the sampler design is known. |
| 207-004 Automatic Sampler Operation and Cut Verification - Final Tails 051-XM-014 | To be set once the sampler design is known. |
| 207-005 Automatic Sampler Cleaning, Unblocking and Cutter Wear Check | To be set once the sampler design is known. |
| 207-006 Automatic versus Manual Sample Bias Check and Reconciliation | Monthly on every sampler, and after any sampler repair or cutter change. |
| 207-007 Cyanide Analyser Daily Operation and Check - 032-CA-001 and 051-CA-002 | To be set once the vendor manual is held. |
| 207-008 Cyanide Analyser Calibration and Standardisation | To be set once the vendor calibration data is held. |
| 207-009 Cyanide Analyser Specialised Electrode Replacement and Conditioning | To be set once the vendor manual is held. |
| 207-010 Cyanide Analyser Sample and Booster Pump Service - 032-PP-207/208 and 041-PP-2xx | To be set once the vendor manual is held. |
| 207-011 Cyanide Analyser Leak Detector and High Level Alarm Response | To be set once the alarm response is defined. |
| 207-012 Analyser to Laboratory Correlation Check and Drift Investigation | Weekly correlation; investigation on any developing drift. |

## Flowchart

Not reproduced here. Each work instruction carries its own step sequence.

# Training

Every person carrying out a task in this area is trained and signed off against the work instruction for that task, and has signed on to the matching JSEA at a communication session.

Site induction, current.

Cyanide awareness and gas detection competency where the area requires it.

Working at height competency where the task requires it.

Competency in the determinations, instruments and calculations used in the task.

Refresher at the interval set by the site training matrix, and after any change to the instruction.

# Document Traceability

The three-digit suffix is shared between the hazard analysis and the work instruction, so SWI-PRO-MET-207-011 is the work instruction for JSEA-PRO-MET-207-011.

| Item | Count |
| --- | --- |
| Tasks in area 207 | 12 |
| Hazard analyses issued | 12 |
| Work instructions issued | 11 |
| Desk-based tasks with a JSEA but no work instruction | 1 |
| Safety-critical | 12 |
| Issued but NOT FOR USE | 10 |

# Review Criteria

This Procedure and the documents beneath it shall be reviewed as follows:

Every 12 months; or

Immediately after any incident, plant change, equipment change or change to the schedule.

# Legislation & References

## Legislation

Queensland Mining and Quarrying Safety and Health Act 1999

Queensland Mining and Quarrying Safety and Health Regulation 2017

## Source Documents

The source documents and notes held for the tasks in this area.

No source held. 022-XM-011 and 022-XM-017 are shop-fabricated with the vendor and cutter design TBA.

The mechanical equipment list shows conflicting status - NEW on one tab, FUTURE on another.

No source held. Vendor and cutter design TBA; status conflict NEW versus FUTURE across the equipment list tabs.

No source held. Vendor and cutter design TBA.

No source held. Vendor and cutter design TBA; no P&ID reference given on the equipment list.

No source held for any of the five samplers.

4034-PR-PRO-002 Sampling Protocol - the manual points to compare against

SWI-PRO-MET-201-001 to -018, the manual sampling instructions, issued

No source held. Molycop Cynoprobe v3 WAD vendor manual not held; OS-040_B Cyanide Analyser still open.

KBK-MIR-MP-PRO-MET-SOP-0053 Pembuatan Larutan Kalium Klorida untuk Cyanide Analizer, received 11 Aug 2026 - the electrolyte preparation only

Molycop Cynoprobe v3 WAD calibration data not held

Electrodes identified as Mintex 3000366 and 3000330 in the instrument list. No procedure held.

Vender Dura 10 pumps are listed as part of the Molycop package. No vendor manual held.

Azbil HPQ-D12 leak detector and E&H FTL31 level switch are vendor scope. No alarm response held.

4034-PR-PRO-002 Sampling Protocol - the laboratory CN methods the analyser must track against

KBK-MIR-MP-PRO-MET-SOP-0033 Penentuan Konsentrasi Sianida dengan Kolorimetri, received 11 Aug 2026

SWI-PRO-MET-201-022 Online Analyser Verification Sampling, issued