Safe Work Instruction

Gold Elution, Electrowinning and Pre-Soak Solution Sampling

This document is the property of Heritage Minerals with all rights are reserved in respect of it. This document cannot be reproduced without the written consent of Heritage Minerals

# Purpose and Scope

This Safe Work Instruction is required so that the mechanical and operational integrity of the system is kept intact. It assists operators and mechanical technicians alike to identify possible problems before they happen.  It will help ensure that when personnel are performing this task it is performed in a safe and secure manner, to ensure the operational integrity of the machine and applies when this task is performed either by itself or in conjunction with other tasks in the area.

A JSA or Take 5 must be completed and discussed with the Supervisor if any variation to this SWI is necessary.

# RESPONSIBILITIES AND AUTHORITY

| What | Who | When |
| --- | --- | --- |
| Conform to procedure: Implement Changes: Change procedure: Review: Authorisation: | All personnel Anyone Training Facilitator Working party of content experts Manager | At all times As required As required On change & yearly On change |

# PPE REQUIREMENTS

- Safety helmet and safety glasses

- Chemical splash goggles and face shield

- High-visibility long-sleeved shirt and long trousers

- Safety boots with sound tread - nitrile PVC boots at the sample point

- Chemical resistant suit, nitrile rubber gloves and apron, heat-resistant gloves

- Personal HCN gas monitor, calibrated and in test date

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Harness and fall arrest where no fixed platform exists

In the event of an emergency

Call on CH19 “EMERGENCY, EMERGENCY, EMERGENCY” on the project to make everyone aware, call 000 for emergency services, police, fire or ambulance, you will need to stay calm and state when requested by a responder:

• Your Name. • Your Location. • Type of Emergency; and • Assistance Required.

| Part 1: Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: |  |  | SWI-PRO-MET-201-013 - Gold Elution, Electrowinning and Pre-Soak Solution Sampling |  |  |  |  |  |  |  |  |  |  |  |
| Issued Date: |  |  | 11/08/2026 |  |  |  |  | Next Review Date: |  |  |  | 11/08/2027 |  |  |
| Document Owner: |  |  | Processing |  |  |  |  | JSEA ID: |  |  |  | JSEA-PRO-MET-201-013 |  |  |
| Part 2: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of work: |  |  |  |  |  | Track the gold through the elution batch - pre-soak, eluate, electrowinning feed and barren electrolyte. The eluate-to-barren-electrolyte pair sets electrowinning efficiency and gold in circuit. Take barren eluate samples every hour through an electrowinning run, for the electrowinning efficiency work in SWI-PRO-MET-203-008 (Step 3). Take a timed series of eluate samples through the elution heating stage (Step 4). Follow the acid wash rinse by pH (Step 5). Area: 061 / 062 Gold recovery. Points and equipment covered: 061-TK-060 - Pre-soak / eluate tank 061-CL-010 - Acid wash column 061-HX-001 / 061-HX-002 - Elution heat exchangers, solution temperature read at sampling 062 - Electrowinning cells, feed and barren electrolyte Frequency: Each elution batch - pre-soak, mid-strip, end-of-strip, and electrowinning feed and barren each shift the cells are running. Electrowinning survey hourly through the run. This instruction sits under Standard Work Procedure HM-PRC-VXX-PRO205 - Plant Sampling and Metallurgical Data Collection. Read that procedure before carrying out this task for the first time. Register line: SWI-PRO-MET-201-013 in HM-MMM-WHS-REG-XXX-R00-SWI-SWMS, tab SWI-MET. The matching hazard analysis is JSEA-PRO-MET-201-013. Draft for review: Steps 3, 4 and 5 are taken from the Martabe work instructions Electrowinning Survey (DOC-3-MET-PRS-WIN-00052-IE), Elution Survey (DOC-3-MET-PRS-WIN-00053-IE) and Acid Wash Survey (DOC-3-MET-PMC-WIN-00107-IE), v1.0, 25/12/2024 and 25/01/2025. They are approved for inclusion. JSEA-PRO-MET-201-013 must be updated for the added hazards before this instruction is approved. Open items: (1) Step 4 samples during the heating stage, which Step 2 does not allow - the JSEA must define the controls; (2) quantity and purpose of the caustic soda in the dark bottles are not stated; (3) which stage is the early stage of an electrowinning run is not stated; (4) the acid wash rinse end-point pH and acid strength are not stated; (5) confirm the HCl respirator and PPE with HSE; (6) confirm the record folders and forms exist at Mt. Morgan. This is a safety-critical task. It is not carried out alone in a cyanide area, and not without the Shift Supervisor knowing it is in progress. |  |  |  |  |  |  |  |  |
| Potential Hazards: |  |  |  |  |  | Hydrogen cyanide gas released at the open sample point — Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. Splash of cyanide-bearing slurry or solution to skin or eyes — Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. Contact with hot solution or hot surface at the elution, stripping or regeneration circuits — Confirm the circuit is not on a heating cycle before sampling. Heat-resistant gloves and face shield. Allow the sample line to flush and cool before collecting. Do not sample a column under pressure. Release from a pressurised sample line, hose or fitting — Confirm the sample line is not blocked before opening. Open slowly and stand clear of the discharge path. Report any weeping fitting rather than tightening it under pressure. Mercury vapour in the gold room, elution and retort areas — Area ventilation confirmed running before entry. Mercury vapour monitoring where the site procedure requires it. No eating, drinking or smoking. Hands washed before leaving the area. Fall from a tank top, launder platform, column platform or access ladder — Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. Contact with an agitator, sample cutter, pump or screen that starts without warning — Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. Slip, trip or fall on wet or slurry-covered walkways around sample points — Spillage cleaned up immediately and the point left clean. Walkways kept clear of hoses, buckets and containers. Safety boots with sound tread. Report any damaged or missing grating before use. Working alone in a cyanide area where a collapse would not be seen — Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. Non-representative sample leading to a wrong process decision — Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. Cross-contamination from a container or cutter not cleaned between points — Bucket, container, cutter and sieve rinsed with process water and then with the stream being sampled, and the rinse discarded, before every sample. Dedicated containers for high-grade streams. Labels checked against the schedule before handover. [JSEA to confirm] HCl gas and HCl liquid at the acid wash sampling points — Respirator mask, rubber gloves, glasses and safety boots. Safety data sheet read. Eyewash proven flowing; wash contact under running water. Spill kit at the point; clean spills at once. The full step-by-step hazard analysis, with the risk ranking, is in JSEA-PRO-MET-201-013 Section 4. |  |  |  |  |  |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT (PPE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part 3: Safe Work Instruction Steps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Job Number (Hazards/Cautions) |  | Application Method |  |  |  |  |  |  | Visual Reference |  |  |  |  |  |
| - Pre-start Check - Working alone in a cyanide area - Gas monitor or fixed detection not healthy - Safety shower or eyewash not proven flowing - Circuit on a heating cycle or column under pressure |  | - Confirm you are trained and signed off against this SWI, signed on to JSEA-PRO-MET-201-013 at a communication session, and authorised by the Shift Supervisor. - Confirm a second person is in the area. Cyanide-area sampling is not done alone. - Confirm with the control room where the batch is in the elution sequence before approaching. - Do NOT sample a column that is under pressure or on a heating cycle. Confirm the circuit is at the sampling condition stated in the elution procedure. - Confirm the plant is in steady-state operation - normal operation for a minimum of 15 to 20 minutes before sampling. A sample taken during a swing is not representative. - Notify the control room or DCS operator that the sampling round is starting, and which points it covers. - Confirm the personal gas monitor is on, in calibration date and reading clean, and that fixed detection in the area is healthy. - Confirm the safety shower and eyewash nearest the point are within reach and proven flowing. - Label every container BEFORE sampling - point, date, time and shift. Equipment Required : - Sample bottles 250 mL, heat-resistant - pH meter - Thermometer or infrared thermometer - Free cyanide titration kit and rhodanine indicator - Filter paper and funnel - 10 bright and 10 dark 250 mL sample bottles per survey, caustic soda, dispatch form - Marcy scale bucket as the sample catch bucket, 20 bright and 20 dark 250 mL bottles per elution survey, plastic sample bags labelled high cyanide - Labelled sample bottles, label paper and pen, calibrated pH meter, respirator and spill kit for HCl - Personal protective equipment as listed in PPE Requirements, including heat-resistant gloves |  |  |  |  |  |  |  |  |  |  |  |  |
| - Elution Batch and Electrowinning Sampling - HCN released at the open sample point - Contact with hot solution - Release from a pressurised sample line - Hot cyanide solution giving a false reading |  | - Flush the sample line and let the first flow discharge to drain until the temperature at the point is steady. - Collect into a heat-resistant bottle held below the point, not in line with the discharge. - Allow the sample to cool before taking pH or titrating - a hot cyanide solution releases HCN faster and gives a false reading. - Bottle and label for gold, silver and copper assay. - Record batch number, elution stage, solution temperature and flow at the time of sampling. CAUTION: Wear heat-resistant gloves and a face shield. Open the valve slowly and stand clear of the discharge path. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Electrowinning Survey - Hourly Barren Eluate - Hot barren eluate splash - Bottle not heat-resistant - Wrong sampling point for the train - Sample not preserved |  | - Take 5 minutes before starting (Take 5). Confirm the controls in Part 2 are in place and the equipment is in good condition. If not, stop and report. Use caustic soda that is in date. - Label each bottle with the strip number, for example barren eluate 3619. Prepare 10 bright and 10 dark 250 mL bottles. - Sample every hour from the early stage to the end. - Take the sample from the correct sampling point. Each electrowinning train has its own sampling point. - Drain the sampling line for about 1 minute so the sample is fresh. Open the tap and fill a heat-resistant bottle held below the point. - Fill the bottle with barren eluate to the top. Repeat for a dark bottle if needed. - Put 1 piece of caustic soda in each dark bottle. - Check every bottle is tightly closed and correctly labelled. Fill the dispatch form for Au, Ag, Cu and free cyanide assay (estimated NaCN 3000 ppm) and send to the Mt. Morgan laboratory. CAUTION: Keep the heat controls of Step 2. Do not stand the bottle on the floor in the discharge path. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Elution Survey - Heating Stage - Hot solution splash at the sample point - Hot cyanide eluate poured to the floor - Hands near the discharge - Samples not traceable to time and temperature |  | - Take 5 minutes before starting (Take 5). Confirm the controls in Part 2 are in place and the 20 bright and 20 dark 250 mL bottles are ready and in good condition. - Take the sample from the sampling point after the basket strainer. Put the Marcy scale bucket under the point to keep hands away from the hot solution. - Open the valve a little and let the solution run for 3 to 4 seconds. Close the valve when the bucket is full. - Let the solution cool, then pour it into a dark bottle and a bright bottle. - Do NOT pour the remaining eluate on the floor. Send it to the cyanide-bearing waste stream and clean the bucket. - Take the first sample when the heating stage reaches 65 degrees C. Record the time and temperature. - Take the second sample when the heating stage reaches 100 degrees C. Record the time and temperature. - Take later samples every 30 minutes, or at the end of the stage. - Check every bottle is tightly closed and correctly labelled. Put all bottles in a sample bag labelled high cyanide. Fill the laboratory form for Au, Ag, Cu and free cyanide assay (estimated NaCN 3000 ppm) and send to the Mt. Morgan laboratory. - Record the elution survey data in G:\Processing\5. Metallurgy\Metallurgy Lab\04 Metlab Data\Elution Survey.xlsx. CAUTION: This survey samples during the heating stage, which Step 2 does not allow. Use only the bucket and heat-resistant bottles under the point, heat-resistant gloves and face shield, a line that is not pressurised, and a second person present. The JSEA must define these controls before use. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Acid Wash Survey - HCl Wash Rinse - HCl gas or liquid at the sample points - HCN released if cyanide is in the acid line - Splash of acid to skin or eyes - pH meter not calibrated |  | - Take 5 minutes before starting (Take 5). Prepare sample bottles, label paper, a pen and a calibrated pH meter. Check everything is in good condition. - Talk to the control room operator about the elution status of each acid wash line. The survey is done during the rinse stage. - Sample from each bucket strainer and from the HCl mixing tank. Drain the sample point line for 30 seconds before sampling. - Pour the sample into a bottle and name it with the train, acid usage, time and sampling point. - Make sure the pH meter is calibrated. If the TPS cube pH meter is used and not calibrated, use SWI-PRO-MET-206-004 Part C. Dip the meter into the sample, wait until the reading is stable and record it. - Keep the data in G:\Processing\5. Metallurgy\MetallurgyLab\04 Metlab Data\Acid Wash Survey or on form DOC-4-MET-PMC-DFR-00134-EN (Form Acid Wash Survey.xlsx). CAUTION: Confirm there is no cyanide in the acid wash line during the rinse - acid on cyanide releases HCN. Keep the personal HCN monitor on. Wear the respirator, rubber gloves, glasses and safety boots, with the eyewash proven flowing. |  |  |  |  |  |  |  |  |  |  |  |  |
| - Completion and Clean Up - Sample valve left weeping - Samples not handed over with a signed record - Control room not told the round is complete |  | - Close the sample valve or stop the sample pump and confirm the point is not weeping. - Wash down the point and the surrounding walkway. Leave nothing on the grating. - Record sampling time, operator name and the operating conditions read at the time - flow, density and any instrument value available. - Deliver the labelled samples to the laboratory or to the composite station as the schedule requires, and sign the handover. - Notify the control room that the round is complete. - Record the result, the method used and the instrument or balance identity on the field or bench data sheet - date, time, shift, operator and every value determined. Make the register or logbook entry and any handover signature. - Record any step not completed, with the reason. - Report to the Shift Supervisor, and record, any control in Part 2 found not in place. |  |  |  |  |  |  |  |  |  |  |  |  |

| Referenced Documents |  |
| --- | --- |
| Document ID | Document Title |
| HM-PRC-VXX-PRO205 | Plant Sampling and Metallurgical Data Collection |
| JSEA-PRO-MET-201-013 | JSEA — Gold Elution, Electrowinning and Pre-Soak Solution Sampling |
| HM-MMM-WHS-REG-XXX-R00-SWI-SWMS | SWI / SWMS Register — tab SWI-MET |
| 4034-PR-PRO-002 | Sampling Protocol, Gold Recovery table |
| 4034-013-VE-028 Rev A | Applied Heat Thermomat 2300 O&M manual (061-BR-001, 061-HE-001, 061-HX-001/002, 061-PP-022, 061-TK-060), received 11 Aug 2026 |
| HM-PRC-V01-PRO010 | Gold Elution Pre-Soak Mixing |
| HM-PRC-V01-PRO011 | Gold Elution |
| HM-PRC-V01-PRO013 | Gold Electrowinning |
| SWI-PRO-MET-203-008 | Elution and Electrowinning Batch Performance Assessment |
| SWI-PRO-MET-206-004 | pH, DO and Conductivity Meter Calibration (Part C, TPS cube) |
| DOC-3-MET-PRS-WIN-00052-IE v1.0 | Martabe WI Electrowinning Survey (25/01/2025) - source of Step 3 |
| DOC-3-MET-PRS-WIN-00053-IE v1.0 | Martabe WI Elution Survey (25/12/2024) - source of Step 4 |
| DOC-3-MET-PMC-WIN-00107-IE v1.0 | Martabe WI Acid Wash Survey (25/12/2024) - source of Step 5 |
| DOC-2-MET-PRM-SOP-00037-IE | Martabe SOP referenced by the three Martabe WIs - not received |
| DOC-2-MET-MEL-SOP-00087-IE | Martabe SOP referenced by the Acid Wash Survey - not received |
| DOC-4-MET-PMC-DFR-00134-EN | Form Acid Wash Survey |

In the event of an emergency

1. HCN Gas Alarm If the personal monitor or fixed detection alarms (5 ppm alarm, 10 ppm high-high), stop work, leave the area upwind and call CH19 “EMERGENCY”. Do not re-enter until the Shift Supervisor clears the area. 2. Contact with Moving Equipment If a person is caught by or struck by an agitator, pump or sample cutter, hit the nearest emergency stop, call CH19 “EMERGENCY” and call 000. Do not approach the equipment until it is isolated and locked out. 3. Suspected Cyanide Exposure If a person becomes dizzy, short of breath or collapses, call CH19 “EMERGENCY”, call 000, and bring the cyanide antidote kit and a trained first aider. Do not give mouth-to-mouth resuscitation. 4. Skin or Eye Contact with Slurry Flush with copious water at the safety shower or eyewash for a minimum of 15 minutes. Remove contaminated clothing while flushing. Report to the first aider. 5. Burns Cool the burn under cool running water for 20 minutes. Do not apply ice. Seek first aid. 6. HCl Gas or Acid Contact Leave the area upwind. Flush skin or eyes with copious water at the safety shower or eyewash for a minimum of 15 minutes. If breathing is affected call CH19 “EMERGENCY” and call 000. 7. Fall from Height Do not move the injured person unless they are in danger. Call CH19 “EMERGENCY”, call 000, and notify the Shift Supervisor. 8. Slip, Fall or Lifting Injury Call CH19 “EMERGENCY”, call 000 if the injury is serious, and notify the Shift Supervisor.