Mt. Morgan SITE WORK INSTRUCTION

Cyanide Destruction (Detox) Bench Test

Standard Work Instruction  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | SWI-PRO-MET-205-015 |
| --- | --- |
| JSEA reference | JSEA-PRO-MET-205-015 |
| Area | 205 Met Lab - Chemical and Leach Testwork |
| Parent procedure | HM-PRC-VXX-PRO209 Metallurgical Laboratory Chemical and Leach Testwork |
| Classification | Safety-critical |
| Revision | Rev A |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

ISSUED BUT NOT FOR USE. Part A (cyanide destruction route test) still cannot be completed - the detox route decision (OR-07) is open, see the folder README. Part B (TSF Toe water treatment screening) has been added in DRAFT from a Martabe work instruction and must be reviewed and approved before use.

# 1  TASK AND SCOPE

Test cyanide destruction at the bench by the candidate routes - peroxide and SO2/air.

The detox route has to be proven at the bench before it is designed, and the tailings cyanide obligation depends on it.

Points and equipment covered:

| Tag / point | Description |
| --- | --- |
| Met lab bench | Physical testwork bench and instrument |

Frequency: To be set once the route is chosen.

This instruction sits under Standard Work Procedure HM-PRC-VXX-PRO209 - Metallurgical Laboratory Chemical and Leach Testwork. Read that procedure before carrying out this task for the first time.

Register line: SWI-PRO-MET-205-015 in HM-MMM-WHS-REG-XXX-R00-SWI-SWMS, tab SWI-MET. The matching hazard analysis is JSEA-PRO-MET-205-015.

# 2  WHO MAY DO THIS

- Trained and signed off against this instruction.

- Signed on to JSEA JSEA-PRO-MET-205-015 at a communication session.

- Site inducted, with current cyanide awareness and gas detection competency where the area requires it.

- Authorised by the Shift Supervisor before the task starts.

This is a safety-critical task. It is not carried out alone in a cyanide area, and not without the Shift Supervisor knowing it is in progress.

# 3  HAZARDS SPECIFIC TO THIS TASK

| Hazard | Control |
| --- | --- |
| Hydrogen cyanide gas released at the open sample point | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. |
| Inhalation of acid fume or reagent vapour at the bench | Work carried out inside the fume cupboard with the sash at the marked working height and the airflow confirmed before starting. Scrubber running and its dosing confirmed. Perchloric work only in the cupboard rated for it, with the washdown routine after every use. No work at an open bench. |
| Contact with sulphuric or hydrochloric acid - burn to skin or eyes | Full acid PPE including face shield, acid-resistant gloves, apron and boots. Sample drawn only from the designated sample valve. Add acid to water, never the reverse. Safety shower and eyewash within reach and proven flowing. Spill kit at the point. |
| Splash of cyanide-bearing slurry or solution to skin or eyes | Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. |
| Cross-contamination from a container or cutter not cleaned between points | Bucket, container, cutter and sieve rinsed with process water and then with the stream being sampled, and the rinse discarded, before every sample. Dedicated containers for high-grade streams. Labels checked against the schedule before handover. |
| Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. |
| [JSEA to confirm] Cyanide-bearing TSF water taken to pH 7 and treated with lime or peroxide | Sample treated as cyanide-bearing: fume cupboard, personal HCN monitor on, full Section 4 PPE, not done alone. pH measured on receipt before the container is opened outside the cupboard. Peroxide handled with face shield and chemical-resistant gloves. All treated water to the cyanide-bearing stream - no general drain. |

The full step-by-step hazard analysis, with the risk ranking, is in JSEA JSEA-PRO-MET-205-015 Section 4.

# 4  PPE

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Acid-resistant gloves, apron and face shield

- Chemical splash goggles

# 5  BEFORE YOU START

Every item below must be true. If one is not, stop and report it.

- Confirm the fume cupboard airflow, the scrubber and its dosing are running before any acid or cyanide work at the bench.

- Confirm the safety shower and eyewash are within reach and proven flowing.

- Confirm the instrument or balance is within its calibration or verification interval.

- Confirm the reagents to be used are in date and correctly labelled.

- Inspect all glassware for chips and cracks and discard anything damaged.

Equipment required:

- Reaction vessel with stirring and pH control

- Peroxide and SO2/air reagents

- Copper catalyst

- pH and ORP meters

- Cyanide analysis set

- Fume cupboard

- [M] Two baffle tanks, agitator or magnetic stirrer, pH meter, camera

- [M] Hydrated lime and hydrogen peroxide; 1000 mL sample bottle for the laboratory

# 6  STEPS

Part A - Cyanide destruction route test (peroxide and SO2/air)

1.  This task cannot be completed. See the folder README for the blocker.

2.  The document is issued so the register line is not empty, and is marked NOT FOR USE.

Part B - TSF Toe to WPP treatment screening: hydrated lime and peroxide [M]

[M] = taken from the Martabe team work instruction "TSF Toe to WPP Test (file name 'Work_Instruction_Tsf_Toe')" (undated). [CONFIRM] = unspecified or inconsistent in the source; resolve before this instruction is approved for use.

Purpose [M]: measure the consumption and effect of peroxide or hydrated lime on TSF Toe water. [CONFIRM] the source steps cover hydrated lime only. No peroxide dose, dosing point, or which baffle tank receives which reagent is stated.

Treat the sample as cyanide-bearing (the laboratory suite includes free, WAD and total CN). Work in the fume cupboard with the personal HCN monitor on. The source lists only safety glasses and rubber gloves, which is below Section 4 - use the Section 4 PPE. [CONFIRM] initial pH of the TSF Toe sample is not stated - measure it first, and do not open any sample whose pH is below 10.5 outside the fume cupboard.

3.  Take 1000 mL of sample as the initial TSF Toe feed. Submit to ITS for Cd, Cu, Fe, Mn, Ni, Zn, Cr, Pb, Hg, TSS, TDS, As, free CN, WAD CN, total CN, nitrite, free ammonia, COD, fluoride and BOD.

4.  Take 2 L of TSF Toe sample and put it into baffle tanks 1 and 2. [CONFIRM] whether 2 L goes into each tank or 2 L is split between them.

5.  Start the agitator or magnetic stirrer.

6.  Measure the initial pH of each sample and record the result.

7.  Add hydrated lime a little at a time.

8.  Record the pH change and the lime added at each addition.

9.  Repeat the addition until the final pH is 7.

10.  Observe the final condition, photograph it and note whether a precipitate has formed.

11.  Submit each sample (1 and 2) to ITS for final pH.

# 7  ON COMPLETION

- Wash down the bench and the fume cupboard working surface.

- Clean glassware and return it to the drying rack.

- Dispose of residues to the correct waste stream - cyanide-bearing effluent does NOT go to the general drain.

- Record the result, the method used and the instrument or balance identity.

# 8  RECORDS

- Field or bench data sheet - date, time, shift, operator, and every value determined.

- Register or logbook entry and any handover signature.

- Any step not completed, recorded with the reason.

- Any control in Section 3 found not in place, reported to the Shift Supervisor and recorded.

# 9  REFERENCE

- OR-07 Back-up Detox definition memo - the decision is open

- Martabe work instruction 'TSF Toe to WPP Test' (Indonesian, undated; file name 'Work_Instruction_Tsf_Toe') - Part B source