Mt. Morgan SITE WORK INSTRUCTION

Adsorption Profile Survey - Metal and Cyanide Resin Circuits

Standard Work Instruction  ·  Heritage Minerals  ·  Mt. Morgan Processing Plant

| Document number | SWI-PRO-MET-203-006 |
| --- | --- |
| JSEA reference | JSEA-PRO-MET-203-006 |
| Area | 203 Plant Surveys |
| Parent procedure | HM-PRC-VXX-PRO207 Plant Surveys and Circuit Performance Assessment |
| Classification | Safety-critical |
| Revision | Rev A |
| Date | 11 August 2026 |
| Prepared by | GGT (cost code 4034-033) |
| Reviewed by | ________________________ |
| Approved by | ________________________ |

# 1  TASK AND SCOPE

Build the profile across the metal and cyanide adsorption trains - solution metal, solution cyanide and resin loading tank by tank.

This is the measure of whether the ReCYN circuit is recovering what it was bought to recover.

Points and equipment covered:

| Tag / point | Description |
| --- | --- |
| 041-TK-007/008 | Metal adsorption train |
| 051-TK-009/010 | Cyanide adsorption train |

Frequency: Quarterly, and after any change to resin inventory or transfer rate.

This instruction sits under Standard Work Procedure HM-PRC-VXX-PRO207 - Plant Surveys and Circuit Performance Assessment. Read that procedure before carrying out this task for the first time.

Register line: SWI-PRO-MET-203-006 in HM-MMM-WHS-REG-XXX-R00-SWI-SWMS, tab SWI-MET. The matching hazard analysis is JSEA-PRO-MET-203-006.

# 2  WHO MAY DO THIS

- Trained and signed off against this instruction.

- Signed on to JSEA JSEA-PRO-MET-203-006 at a communication session.

- Site inducted, with current cyanide awareness and gas detection competency where the area requires it.

- Authorised by the Shift Supervisor before the task starts.

This is a safety-critical task. It is not carried out alone in a cyanide area, and not without the Shift Supervisor knowing it is in progress.

# 3  HAZARDS SPECIFIC TO THIS TASK

| Hazard | Control |
| --- | --- |
| Hydrogen cyanide gas released at the open sample point | Personal HCN monitor worn and switched on, in calibration date. Fixed detection confirmed healthy - alarm 5 ppm, high-high 10 ppm. Sample point approached from upwind. pH of the stream confirmed above 10.5 before opening. Second person in sight and clear. Cyanide antidote kit on site and a trained first aider on shift. Withdraw upwind and report on any reading above the site action level. |
| Splash of cyanide-bearing slurry or solution to skin or eyes | Sample valve opened slowly with the container positioned first and the body out of the spray path. Face shield, chemical splash goggles, chemical-resistant gloves and apron. Never open a sample valve whose discharge cannot be seen. Safety shower and eyewash proven flowing before the round starts. |
| Fall from a tank top, launder platform, column platform or access ladder | Fixed platforms, stairways and handrails used in preference to any other access. Harness and fall arrest where no fixed platform exists. Three points of contact on ladders. No sampling at height alone. Sample containers carried in a bag or hoisted, not in the hand on a ladder. |
| Contact with an agitator, sample cutter, pump or screen that starts without warning | Much of the plant is automated and equipment can start or stop without warning. Guards confirmed in place before approaching. Control room notified before the round starts and again on completion. Isolation and lock-out before any guard or cutter housing is opened. |
| Working alone in a cyanide area where a collapse would not be seen | Cyanide-area sampling is not done alone. Control room notified at the start and end of every round. Radio check-in at the interval set by the Shift Supervisor. |
| Slip, trip or fall on wet or slurry-covered walkways around sample points | Spillage cleaned up immediately and the point left clean. Walkways kept clear of hoses, buckets and containers. Safety boots with sound tread. Report any damaged or missing grating before use. |
| Non-representative sample leading to a wrong process decision | Line flushed for the stated time before collecting. Sample taken at the same point, in the same way and at the same frequency every time. Container labelled BEFORE the sample is taken, with point, date, time and shift. Steady-state confirmed for 15 to 20 minutes before sampling. A sample taken off-condition is marked as such or discarded. |
| Cross-contamination from a container or cutter not cleaned between points | Bucket, container, cutter and sieve rinsed with process water and then with the stream being sampled, and the rinse discarded, before every sample. Dedicated containers for high-grade streams. Labels checked against the schedule before handover. |

The full step-by-step hazard analysis, with the risk ranking, is in JSEA JSEA-PRO-MET-203-006 Section 4.

# 4  PPE

- Hard hat

- Safety glasses

- High-visibility long-sleeved shirt and long trousers

- Safety boots

- Gloves appropriate to the task

- Personal HCN gas monitor, calibrated and in test date

- Chemical splash goggles and face shield

- Nitrile rubber gloves

- Nitrile PVC boots

- Chemical resistant suit

- Full or half gas mask with A2B2E2K2P3 cartridge, carried and available for immediate use

- Harness and fall arrest where no fixed platform exists

# 5  BEFORE YOU START

Every item below must be true. If one is not, stop and report it.

- Notify the control room that the task is starting, and where you will be.

- Confirm the personal gas monitor is on, in calibration date and reading clean.

- Confirm the safety shower and eyewash nearest the work are within reach and proven flowing.

- Confirm the area is fit - no restricted access, no gas alarm, no spill response in progress.

Equipment required:

- Sample containers pre-labelled by tank

- pH meter

- Free cyanide titration kit

- 0.9 mm sieve

- Measuring glass

- Filter press and filter paper

- Field data sheets

# 6  STEPS

1.  Confirm steady state and hold the circuit through the survey.

2.  Sample every adsorption tank in sequence in one pass.

3.  Take resin concentration and a resin sample from each tank on the same pass.

4.  Determine pH and free cyanide in the field.

5.  Submit solution for metal assay and resin for loading assay.

6.  Plot solution metal, solution cyanide and resin loading down each train.

7.  Compare the measured profile against the design adsorption isotherm and report the deviation.

8.  Check the bead size distribution on the resin samples as an attrition indicator.

# 7  ON COMPLETION

- Leave the area clean. Recover spillage to the drain, not to the ground.

- Return all equipment to its storage point.

- Record what was done, what was found and anything not completed.

- Notify the control room that the task is complete.

# 8  RECORDS

- Field or bench data sheet - date, time, shift, operator, and every value determined.

- Register or logbook entry and any handover signature.

- Any step not completed, recorded with the reason.

- Any control in Section 3 found not in place, reported to the Shift Supervisor and recorded.

# 9  REFERENCE

- 4034-PR-PRO-002 Sampling Protocol, ReCYN Adsorption table

- KBK-MIR-MP-PRO-MET-SOP-0049 resin tank sampling and profile and SOP-0058 resin PSD, both received 11 Aug 2026

- KBK SOP-PROC-CNREC-01 Resin transfer between adsorption tanks, received 11 Aug 2026

- HM-PRC-V01-PRO006 and PRO007